NitrOS-9 Technical Reference

The NitrOS-9 Project

http://www.nitros9.org

Revision History:

| Revision | Date | Comment |
| --- | --- | --- |
| 0.1 | July 10, 2004 | Created |

Acknowledgements:

Input and Typesetting: Boisy Pitre.

Table of Contents

NitrOS-9 Technical Reference	1

Chapter 1.	System Organization	6

Layer 1: The Kernel	6

Layer 2: IOMan	7

Layer 3: File Managers	7

Layer 4: Device Drivers	7

Layer 5: Device Descriptors	7

Beyond Layer 5: Applications	8

Chapter 2.	The Kernel	9

System Initialization	10

System Call Processing	10

OS9Defs and Symbolic Names	10

Types of System Calls	11

Memory Management	11

Memory Use in NitrOS-9	12

Color Computer 3 Memory Management Hardware	13

Multiprogramming	17

Process Creation	17

Process States	18

Execution Scheduling	19

Signals	19

Interrupt Processing	20

Logical Interrupt Polling System	21

Virtual Interrupt Processing	23

Chapter 3.	Memory Modules	26

Module Types	26

Module Format	26

Module Header	27

Module Body	27

CRC Value	27

Module Headers: Standard Information	28

Sync Bytes	28

Module Size	28

Offset to Module Name	28

Type/Language Byte	28

Attributes/Revision Level Byte	29

Header Check	29

Module Headers: Type-Dependent Information	29

Chapter 4.	NitrOS-9’s Unified Input/Output System	33

The I/O Manager	34

File Managers	34

File Manager Structure	35

Create, Open	35

MakDir	35

ChgDir	35

Delete	36

Seek	36

Read	36

Write	36

ReadLn	37

WriteLn	37

GetStat, PutStat	37

Close	37

Interfacing with Device Drivers	37

Device Driver Modules	38

NitrOS-9 Interaction with Devices	41

Suspend State (NitrOS-9 Level 2 only)	43

Device Descriptor Modules	44

Path Descriptors	46

Chapter 5.	Random Block File Manager	48

Logical and Physical Disk Organization	48

Identification Sector (LSN 0)	49

Disk Allocation Map Sector (LSN 1)	49

Root Directory	50

File Descriptor Sector	50

Directory	52

The RBF Manager Definitions of the Path Descriptor	52

RBF-Type Device Descriptor Modules	54

RBF Record Locking	55

Record Locking and Unlocking	55

Non-Shareable Files	56

End-of-File Lock	57

Deadlock Detection	57

RBF-Type Device Driver Modules	57

The RBF Device Memory Area Definitions	58

RBF Device Driver Subroutines	60

Chapter 6.	Sequential Character File Manager	68

SCF Line Editing Functions	68

Read and Write	68

Read Line and Write Line	69

SCF Definitions of the Path Descriptor	69

SCF-Type Device Descriptor Modules	72

SCF-Type Device Driver Modules	74

SCF Device Driver Subroutines	75

Chapter 7.	The Pipe File Manager (PIPEMAN)	83

Chapter 8.	System Calls	86

Calling Procedure	86

I/O System Calls	87

System Call Descriptions	87

User Mode System Calls Quick Reference	88

System Mode Calls Quick Reference	89

User System Calls	90

I/O User System Calls	91

Privileged System Mode Calls	91

Get Status System Calls	91

Set Status System Calls	91

Chapter 9.	Appendices	92

A. System Module Diagrams	92

B. Standard Floppy Disk Format	92

C. System Error Codes	92

# System Organization

The NitrOS-9 Operating System is composed of groups of modules that work together to perform a common task. The following illustration shows the major modules and their position in the five-layer organization of NitrOS-9:

NitrOS-9 Kernel (Krn, KrnP2)

Input/Output Manager (IOMan)

Disk File Manager (RBF)

Character File Manager (SCF)

Pipe File Manager (PIPEMAN)

Floppy Driver (rb1773)

Terminal or Serial Driver

Pipe Driver (Piper)

D0

D1

D2

Term

Pipe

T1

T2

Init

Clock

Clock2

Layer 5

Layer 4

Layer 3

Layer 2

Layer 1

## Layer 1: The Kernel

At the lowest layer, Krn and KrnP2 make up the two primary parts of the kernel, or core of NitrOS-9. It is the kernel that provides the intelligence behind NitrOS-9, and handles basic system services such as multitasking and memory management. The kernel also links all other NitrOS-9 modules into the system.

Another important set of modules that reside at this layer are Clock and Clock2. Together, these two modules work to keep track of both system time (known as the tick, the heartbeat of the system) as well as actual clock time, either through software or via real-time clock hardware.

The final module of this layer is Init. This module contains a table of initialization values and is consulted by the kernel during system startup. Information such as the user task to run after boot, initial table sizes, and device names are found in this module. It is loaded into RAM (random access memory) by the NitrOS-9 bootstrap module Boot, along with other necessary system modules.

## Layer 2: IOMan

The system’s second layer (just above kernel) contains the input/output manager, IOMan. This module provides common processing for all input/output operations, and is required for performing any I/O supported by NitrOS-9.

## Layer 3: File Managers

The system’s third layer contains file managers. File managers perform I/O request processing for similar classes of I/O devices. There are three file managers:

RBF	The random block file manager processes all disk I/O operations.

SCF	The sequential character file manager handles all non-disk I/O operations that operate one character at a time. These operations include terminal and printer I/O.

PIPEMAN	The pipe file manager handles pipes. Pipes are memory buffers that act as files. Pipes are used for data transfers between processes.

## Layer 4: Device Drivers

The system’s fourth layer is the device driver layer. Device drivers handle basic I/O functions for specific I/O controller hardware, and are normally provided to you when you purchase new I/O devices or cartridges. You can use pre-written drivers, or you can write your own.

## Layer 5: Device Descriptors

The system’s fifth layer contains the device descriptors. Device descriptors are small tables that define the logical name, device driver and file manager for each I/O port. They also contain port initialization and port address information. Device descriptors require only one copy of each I/O controller driver used.

## Beyond Layer 5: Applications

NitrOS-9’s primary purpose is to act as the manager of data flow for applications, which run as processes outside of the five layer hierarchy. This includes the initial user process, SysGo, which is forked after boot, and Shell, the program that allows commands to be typed and executed by NitrOS-9.

# The Kernel

The kernel, as stated in the previous chapter, is the true core of NitrOS-9. All resource management and services, from memory allocation to the creation and destruction of processes, are supervised by this very important software component.

The kernel is actually split into two parts: Krn (which holds core system calls that must be present during the boot process) and KrnP2 (which handles additional system calls). These two modules complete the concept of the NitrOS-9 kernel.

The kernel modules for NitrOS-9 Level 1 are smaller than those of NitrOS-9 Level 2, and are small enough to reside on the boot track. Under NitrOS-9 Level 2, Krn resides in the boot track while KrnP2 is part of the OS9Boot file, a file that is loaded into RAM with the other NitrOS-9 modules at bootstrap time.

Here’s a look at the kernel’s main responsibilities:

- System initialization after reset

- Service request processing

- Memory management

- Multiprogramming management

- Interrupt processing

I/O functions are not included in the list because the kernel does not directly process them. Instead, it passes I/O system calls to the I/O Manager, IOMan, for processing.

We will now explore the kernel’s responsibilities in more detail.

## System Initialization

After a hardware reset, the kernel initializes the system. This involves:

- Locating modules loaded into memory from the NitrOS-9 boot file.

- Determining the amount of available RAM.

- Loading any required modules that were not loaded from the NitrOS-9 boot file.

NitrOS-9 also adds the ability to install new system calls through the F$SSvc system service call. Under NitrOS-9 Level 1, user state programs can directly call this system call. However, NitrOS-9 Level 2 user processes cannot call this system call directly because it is privileged. Instead, new system calls are added through special kernel extension modules, named KrnP3, KrnP4, KrnP5, etc. These kernel modules must be present in the OS9Boot file. The cold start routine in KrnP2 performs a link to KrnP3, and if it exists in the boot file, it will be branched to. If KrnP3 does not exist in the boot file, KrnP2 continues with a normal cold start.

## System Call Processing

System Calls are used to communicate between NitrOS-9 and programs for such functions as memory allocation and process creation. In addition to I/O and memory management functions, system calls have other functions. These include inter-process control and timekeeping.

System calls use the 6809 microprocessor’s SWI2 instruction followed by a constant byte representing the code. You usually pass parameters for system calls in the 6809 registers.

### OS9Defs and Symbolic Names

A system-wide assembly language equate file, called OS9Defs, defines symbolic names for all system calls. This file is normally included when assembling hand-written or compiler-generated code. The NitrOS-9 assembler has a built-in macro to generate system calls. For example:

os9 I$Read

is recognized and assembled as equivalent to:

swi2

fcb	I$Read

The NitrOS-9 assembler macro “os9” generates an SWI2 instruction. The label I$Read is the label for the system call code $89.

### Types of System Calls

System calls are divided into two categories: I/O calls and function calls.

I/O calls perform various input/output functions. The kernel passes calls of this type to the I/O manager for processing. The symbolic names for I/O calls begin with I$ instead of F$. For example, the Read system call is called I$Read.

Function calls perform memory management, multi-programming and other functions, with most being processed by the kernel. The symbolic names for function calls begin with F$. For example, the Link function call is called F$Link.

The function calls include user calls and privileged system mode calls. (See Chapter 8, “System Calls,” for more information.)

## Memory Management

Memory management is an important operating system function. Using memory and modules, NitrOS-9 manages the logical contents of memory and the physical assignment of memory to programs.

An important concept in memory management is the memory module. The memory module is a format in which programs must reside. NitrOS-9 maintains a module directory that points to the modules that occupy memory. This module directory contains information about each module, including its name and address and the number of processes using it. The number of processes using a module is reflected in the module’s link count.

When a module’s link count reaches zero, NitrOS-9 releases the module, returns the memory it held back to the free pool, and removes its name from the module directory.

Memory modules are the foundation of NitrOS-9’s modular software environment, and have several advantages:

- Automatic runtime linking of programs to libraries of utility modules

- Automatic sharing of re-entrant programs

- Replacement of small sections of large programs into memory for update or correction.

### Memory Use in NitrOS-9

NitrOS-9 automatically allocates memory when any of the following occurs:

- Program modules are loaded into RAM

- Processes are created

- Processes execute system calls to request additional RAM

- NitrOS-9 needs I/O buffers or larger tables

NitrOS-9 also has inverse functions to deallocate memory allocated to program modules, new processes, buffers, and tables.

In general, memory for program modules and buffers is allocated from high addresses downward. Memory for process data areas is allocated from low addresses upward.

#### NitrOS-9 Level 1 Memory Specifics

Under NitrOS-9 Level 1, a maximum of 64K of RAM is supported. The operating system and all processes must share this memory. In the 64K address map, NitrOS-9 reserves some space at the top and bottom of RAM for its own use. The amount depends on the sizes of system tables that are specified in the Init module.

NitrOS-9 pools all other RAM into a free memory space. As the system allocates or deallocates memory, it dynamically takes it from or returns it to this pool. Under NitrOS-9 Level 2, RAM does not need to be contiguous because the memory management unit can dynamically rearrange memory addresses.

The basic unit of memory allocation is the 256-byte page. NitrOS-9 Level 1 always allocates memory in whole numbers of pages.

The data structure that NitrOS-9 uses to keep track of memory allocation is a 256-byte bitmap. Each bit in this table is associated with a specific page of memory. A cleared bit indicates that the page is free and available for assignment. A set bit indicates that the page is in use (that no RAM is free at that address).

#### NitrOS-9 Level 2 Memory Specifics

Because NitrOS-9 Level 2 utilizes the Memory Management Unit (MMU) component of the Color Computer 3, up to 2MB of memory can be supported. However, each process is still limited to a maximum of 64K of RAM.

Even with this limitation, there is a significant advantage over NitrOS-9 Level 1. Every process has its own 64K “playground.” Even the operating system itself has its own 64K area. This means that programs do not have to share a single 64K block with each other or the system. Consequently, larger programs are possible under NitrOS-9 Level 2.

These 64K areas are made up of 8K blocks, the size that is imposed by the MMU found in the Color Computer 3. NitrOS-9 Level 2 assembles a number of these 8K blocks to provide every process (including the system) its own 64K working area.

Within the system’s 64K address map, memory is still allocated in 256-byte pages, just like NitrOS-9 Level 1.

### Color Computer 3 Memory Management Hardware

As mentioned previously, the 8-bit CPU in the Color Computer 3 can directly address only 64K of memory. This limitation is imposed by the 6809, which has only 16 address lines (A0-A15). The Color Computer 3’s Memory Management Unit (MMU) extends the addressing capability of the computer by increasing the address lines to 19 (A0-A18). This lets the computer address up to 512K of memory ($0-$7FFFF), or up to 2MB of memory ($0-$1FFFFF) when enhanced with certain memory upgrades. In this document we will discuss the more common 512K configuration.

The 512K address space is called the physical address space. The physical address space is subdivided into 8K blocks. The six high order address bits (A13-A18) define a block number.

NitrOS-9 creates a logical address space of up to 64K for each task by using the F$Fork system call. Even though the memory within a logical address space appears to be contiguous, it might not be—the MMU translates the physical addresses to access available memory. Address spaces can also contain blocks of memory that are common to more than one map.

The MMU consists of a multiplexer and a 16 by 6-bit RAM array. Each of the 6-bit elements in this array is an MMU task register. The computer uses these task registers to determine the proper 8-kilobyte memory segment to address.

The MMU task registers are loaded with addressing data by the CPU. This data indicates the actual location of each 8-kilobyte segment of the current system memory. The task register are divided into two sets consisting of eight registers each. Whether the task register select bit (TR bit) is set or reset determines which of the two sets is to be used.

The relation between the data in the task register and the generated addresses is as follows:

| Bit | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- |
| Corresponding Memory Address | A18 | A17 | A16 | A15 | A14 | A13 |

When the CPU accesses any memory outside the I/O and control range (XFF00-XFFFF), the CPU address lines (A13-A15) and the TR bit determine what segment of memory to address. This is done through the multiplexer when SELECT is low (See the following table.)

When the CPU writes data to the MMU, A0-A3 determine the location of the MMU register to receive the incoming data when SELECT is high. The following diagram illustrates the operation of the Color Computer 3’s memory management.

The system uses the data from the MMU registers to determine the block of memory to be accessed, according to the following table:

| TR Bit | A15 A14 A13 | Address Range | MMU Address |
| --- | --- | --- | --- |
| 0 0 0 0 0 0 0 0 | 0 0 0 0 0 1 0 1 0 0 1 1 1 0 0 1 0 1 1 1 0 1 1 1 | X0000-X1FFF X2000-X3FFF X4000-X5FFF X6000-X7FFF X8000-X9FFF XA000-XBFFF XC000-XDFFF XE000-XFFFF | FFA0 FFA1 FFA2 FFA3 FFA4 FFA5 FFA6 FFA7 |
| 1 1 1 1 1 1 1 1 | 0 0 0 0 0 1 0 1 0 0 1 1 1 0 0 1 0 1 1 1 0 1 1 1 | X0000-X1FFF X2000-X3FFF X4000-X5FFF X6000-X7FFF X8000-X9FFF XA000-XBFFF XC000-XDFFF XE000-XFFFF | FFA8 FFA9 FFAA FFAB FFAC FFAD FFAE FFAF |

The translation of physical addresses to 8K blocks is as follows:

| Range From To | Block Number |  | Range From To | Block Number |
| --- | --- | --- | --- | --- |
| 00000 01FFF | 00 |  | 40000 41FFF | 20 |
| 02000 03FFF | 01 |  | 42000 43FFF | 21 |
| 04000 05FFF | 02 |  | 44000 45FFF | 22 |
| 06000 07FFF | 03 |  | 46000 47FFF | 23 |
| 08000 09FFF | 04 |  | 48000 49FFF | 24 |
| 0A000 0BFFF | 05 |  | 4A000 4BFFF | 25 |
| 0C000 0DFFF | 06 |  | 4C000 4DFFF | 26 |
| 0E000 0FFFF | 07 |  | 4E000 4FFFF | 27 |
| 10000 11FFF | 08 |  | 50000 51FFF | 28 |
| 12000 13FFF | 09 |  | 52000 53FFF | 29 |
| 14000 15FFF | 0A |  | 54000 55FFF | 2A |
| 16000 17FFF | 0B |  | 56000 57FFF | 2B |
| 18000 19FFF | 0C |  | 58000 59FFF | 2C |
| 1A000 1BFFF | 0D |  | 5A000 5BFFF | 2D |
| 1C000 1DFFF | 0E |  | 5C000 5DFFF | 2E |
| 1E000 1FFFF | 0F |  | 5E000 5FFFF | 2F |
| 20000 21FFF | 10 |  | 60000 61FFF | 30 |
| 22000 23FFF | 11 |  | 62000 63FFF | 31 |
| 24000 25FFF | 12 |  | 64000 65FFF | 32 |
| 26000 27FFF | 13 |  | 66000 67FFF | 33 |
| 28000 29FFF | 14 |  | 68000 69FFF | 34 |
| 2A000 2BFFF | 15 |  | 6A000 6BFFF | 35 |
| 2C000 2DFFF | 16 |  | 6C000 6DFFF | 36 |
| 2E000 2FFFF | 17 |  | 6E000 6FFFF | 37 |
| 30000 31FFF | 18 |  | 70000 71FFF | 38 |
| 32000 33FFF | 19 |  | 72000 73FFF | 39 |
| 34000 35FFF | 1A |  | 74000 75FFF | 3A |
| 36000 37FFF | 1B |  | 76000 77FFF | 3B |
| 38000 39FFF | 1C |  | 78000 79FFF | 3C |
| 3A000 3BFFF | 1D |  | 7A000 7BFFF | 3D |
| 3C000 3DFFF | 1E |  | 7C000 7DFFF | 3E |
| 3E000 3FFFF | 1F |  | 7E000 7FFFF | 3F |

In order for the MMU to function, the TR bit at $FF90 must be cleared and the MMU must be enabled. However, before doing this, the address data for each memory segment must be loaded into the designated set of task registers. For example, to select a standard 64K map in the top range of the Color Computer 3’s 512K RAM, with the TR bit set to 0, the following values must be preloaded into the MMU’s registers:

| MMU Location Address | Data (Hex) | Data (Binary) | Address Range |
| --- | --- | --- | --- |
| FFA0 FFA1 FFA2 FFA3 FFA4 FFA5 FFA6 FFA6 | 38 39 3A 3B 3C 3D 3E 3F | 111000 111001 111010 111011 111100 111101 111110 111111 | 70000-71FFF 72000-73FFF 74000-75FFF 76000-77FFF 78000-79FFF 7A000-7BFFF 7C000-7DFFF 7E000-7FFFF |

Although this table shows MMU data in the range $38 to $3F, any data between $0 and $3F can be loaded into the MMU registers to select memory addresses in the range 0 to $7FFFF.

Normally, the blocks containing I/O devices are kept in the system map, but not in the user maps. This is appropriate for timesharing applications, but not for process control. To directly access I/O devices, use the F$MapBlk system call. This call takes a starting block number and block count, and maps them into unallocated spaces of the process’ address space. The system call returns the logical address at which the blocks were inserted.

For example, suppose a display screen in your system is allocated at extended addresses $7A000-$7DFFF (blocks $3D and $3E). The following system call maps them into your address space:

ldb 		#$02 		number of blocks

ldx 		#$3D 		starting block number

os9 		F$MapBlk 	call MapBlk

stu 		IOPorts 		save address where mapped

On return, the U register contains the starting address at which the blocks were switched. For example, suppose that the call returned $4000. To access extended address $7A020, write to $4020.

Other system calls that copy data to or from one task’s map to another are available, such as F$STABX and F$Move. Some of these calls are system mode privileged. You can unprotect them by changing the appropriate bit in the corresponding entry of the system service request table and them making a new system boot with the patched table.

## Multiprogramming

NitrOS-9 is a multiprogramming operating system. This means that several independent programs called processes can be executed at the same time. By issuing the appropriate system call to NitrOS-9, each process can have access to any system resource.

Multiprogramming functions use a hardware real-time clock. The clock generates interrupts 60 times per second, or one every 16.67 milliseconds. These interrupts are called ticks.

Processes that are not waiting for some event are called active processes. NitrOS-9 runs active processes for a specific system-assigned period called a time slice. The number of time slices per minute during which a process is allowed to execute depends on a process’ priority relative to all other active processes. Many NitrOS-9 system calls are available to create, terminate and control processes.

### Process Creation

A process is created when an existing process executes the F$Fork system call. This call’s main argument is the name of the program module that the new process is to execute first (the primary module).

Finding the Module. NitrOS-9 first attempts to find the module in the module directory. If it does not find the module, NitrOS-9 usually attempts to load into a memory a mass-storage file in the execution directory, with the requested module name as a filename.

Assigning a Process Descriptor. Once OS-9 finds the module, it assigns the process a data structure called a process descriptor. This is a 64-byte package that contains information about the process, its state (see the following section, “Process States”), memory allocations, priority, queue pointers, and so on. NitrOS-9 automatically initializes and maintains the process descriptor.

Allocate RAM. The next step is to allocate RAM for the process. The primary module’s header contains a storage size, which NitrOS-9 uses, unless a larger one was requested at fork time. The memory is allocated from the free memory space and given to that process.

Assign Process ID and User ID. NitrOS-9 assigns the new process a unique number called a process ID. Other processes can communicate with the process by referring to its ID in various system calls.

The process also has a user ID, which is used to identify all processes and files that belong to a particular user. The user ID is inherited from the parent process.

Process Termination. A process terminates when it executes the F$Exit system call, or when it receives a fatal signal. The termination closes any open paths, deallocates memory used by the process, and unlinks its primary module.

### Process States

At any instant a process can be in one of three states:

- Active – The process is ready for execution.

- Waiting – The process is suspended until a child process terminates or until it receives a signal. A child process is a process that is started by another process known as the parent process.

- Sleeping – The process is suspended for a specific period of time or until it receives a signal.

Each state has its own queue, a linked list of descriptors of processes in that state. To change a process’ state, NitrOS-9 moves its descriptor to another queue.

The Active State. Each active process is given a time slice for execution, according to its priority. The scheduler in the kernel ensures that all active processes, even those of low priority, get some CPU time.

The Wait State. This state is entered when a process executes the F$Wait system call. The process remains suspended until one of its child processes terminates or until it receives a signal. (See the “Signals” section later in this chapter.)

The Sleep State. This state is entered when a process executes the F$Sleep system call, which expects the number of ticks for which the process is to remain in the sleep queue. The process will remain until the specified time has elapsed, or until it receives a wakeup signal.

### Execution Scheduling

The NitrOS-9 scheduler uses an algorithm that ensures that all active processes get some amount of execution time.

All active processes are members of the active process queue, which is kept sorted by process age. Age is the number of process switches that have occurred since the process’ last time slice. When a process is moved to the active process queue from another queue, its age is set according to its priority—the higher the priority, the higher the age.

Whenever a new process becomes active, the ages of all other active processes increase by one time slice count. When the executing process’ time slice has elapsed, the scheduler selects the next process to be executed (the one with the next highest age, the first one in the queue). At this time, the ages of all other active processes increase by one. Ages never go beyond 255.

A new active process that was terminated while in the system state is an exception. The process is given high priority because it is usually executing critical routines that affect shared system resources.

When there are no active processes, the kernel handles the next interrupt and then executes a CWAI instruction. This procedure decreases interrupt latency time (the time it takes the system to process an interrupt).

### Signals

A signal is an asynchronous control mechanism used for interprocess communication and control. It behaves like a software interrupt, and can cause a process to suspend a program, execute a specific routine, and then return to the interrupted program.

Signals can be sent from one process to another by the F$Send system call. Or, they can be sent from NitrOS-9 service routines to a process.

A signal can convey status information in the form of a 1-byte numeric value. Some signal codes (values) are predefined, but you can define most. Those already defined by NitrOS-9 are:

| 0 | Kill (terminates the process, is non-interceptable) |
| --- | --- |
| 1 | Wakeup (wakes up a sleeping process) |
| 2 | Keyboard terminate |
| 3 | Keyboard interrupt |
| 4 | Window change |
| 128-255 | User defined |

When a signal is sent to a process, the signal is saved in the process descriptor. If the process is in the sleeping or waiting state, it is changed to the active state. When the process gets its next time slice, the signal is processed.

What happens next depends on whether or not the process has set up a signal intercept trap (also known as a signal service routine) by executing the F$Icpt system call.

If the process has set up a signal intercept trap, the process resumes execution at the address given in the system call. The signal code passes to this routine. Terminate the routine with an RTI instruction to resume normal execution of the process.

Note: A wakeup signal activates a sleeping process. It sets a flag but ignores the call to branch to the intercept routine.

If it has not set up a signal intercept trap, the process is terminated immediately. It is also terminated if the signal code is zero. If the process is in the system mode, NitrOS-9 defers the termination. The process dies upon return to the user state.

A process can have a signal pending (usually because the process has not been assigned a time slice since receiving the signal). If it does, and another process tries to send it another signal, the new signal is terminated, and the F$Send system call returns an error. To give the destination process time to process the pending signal, the sender needs to execute an F$Sleep system call for a few ticks before trying to send the signal again.

## Interrupt Processing

Interrupt processing is another important function of the kernel. OS-9 sends each hardware interrupt to a specific address. This address, in turn, specifies the address of the device service routine to be executed. This is called vectoring the interrupt. The address that points to the routine is called the vector. It has the same name as the interrupt.

The SWI, SWI2, and SWI3 vectors point to routines that read the corresponding pseudo vector from the process’ descriptor and dispatch to it. This is why the F$SSWI system call is local to a process; it only changes a pseudo vector in the process descriptor.

| Vector | Address |
| --- | --- |
| SWI3 | $FFF2 |
| SWI2 | $FFF4 |
| FIRQ | $FFF6 |
| IRQ | $FFF8 |
| SWI | $FFFA |
| NMI | $FFFC |
| RESTART | $FFFE |

FIRQ Interrupt. The system uses the FIRQ interrupt. The FIRQ vector is not available to you. The FIRQ vector is reserved for future use. Only one FIRQ generating device can be in the system at a time.

### Logical Interrupt Polling System

Because most NitrOS-9 I/O devices use IRQ interrupts, NitrOS-9 includes a sophisticated polling system. The IRQ polling system automatically identifies the source of the interrupt, and then executes its associated user- or system-defined service routine.

IRQ Interrupt. Most NitrOS-9 I/O devices generate IRQ interrupts. The IRQ vector points to the real-time clock and the keyboard scanner routines. These routines, in turn, jump to a special IRQ polling system that determines the source of the interrupt. The polling system is discussed in an upcoming paragraph.

NMI Interrupt. The system uses the NMI interrupt. The NMI vector, which points to the disk driver interrupt service routine, is not available to you.

The Polling Table. The information required for IRQ polling is maintained in a data structure called the IRQ polling table. The table has an entry for each device that might generate an IRQ interrupt. The table size is permanent and is defined by an initialization constant in the Init module. Each entry in the polling table is given a number from 0 (lowest priority) to 255 (highest priority). In this way, the more important devices (those that have a higher interrupt frequency) can be polled before the less important ones.

Each entry has six variables:

| Polling Address | Points to the status register of the device. The register must have a bit or bits that indicate if it is the source of an interrupt. |
| --- | --- |
| Flip byte | Selects whether the bits in the device status register indicate active when set or active when cleared. If a bit in the flip byte is set, it indicates that the task is active whenever the corresponding bit in the status register is clear. |
| Mask Byte | Selects one or more interrupt request flag bits within the device status register. The bits identify the active task or device. |
| Service Routine Address | Points to the interrupt service routine for the device. You supply this address. |
| Static Storage Address | Points to the permanent storage area required by the device service routine. You supply this address. |
| Priority | Sets the order in which the devices are polled (a number from 0 to 255). |

Polling the Entries. When an IRQ interrupt occurs, NitrOS-9 enters the polling system via the corresponding RAM interrupt vector. It starts polling the devices in order of priority. NitrOS-9 loads the status register address of each entry into Accumulator A, using the device address from the table.

NitrOS-9 performs an exclusive-OR operation using the flip byte, followed by a logical-AND operation using the mask byte. If the result is non-zero, NitrOS-9 assumes that the device is the source of the interrupt.

NitrOS-9 reads the device memory address and service routine address from the table, and performs the interrupt service routine.

Note: If you are writing your own device driver, terminate the interrupt service routine with an RTS instruction, not an RTI instruction.

Adding Entries to the Table. You can make entries to the IRQ (interrupt request) polling table by using the F$IRQ system call. This call is a privileged system call, and can only be executed in system mode. NitrOS-9 is in system mode whenever it is running a device driver.

Note: The code for the interrupt polling system is located in the I/O Manager module. The Krn and KrnP2 modules contain the physical interrupt processing routines.

## Virtual Interrupt Processing

A virtual IRQ, or VIRQ, is useful with devices in Multi-Pak expansion slots. Because of the absence of an IRQ line from the Multi-Pak interface, these devices cannot initiate physical interrupts. VIRQ enables these devices to act as if they were interrupt drive. Use VIRQ only with device driver and pseudo device driver modules. VIRQ is handled in the Clock module, which handles the VIRQ polling table and installs the F$VIRQ system call. Since the F$VIRQ system call is dependent on clock initialization, the SysGo module forces the clock to start.

The virtual interrupt is set up so that a device can be interrupted at a given number of clock ticks. The interrupt can occur one time, or can be repeated as long as the device is used.

The F$VIRQ system call installs VIRQ in a table. This call requires specification of a 5-byte packet for use in the VIRQ table. This packet contains:

- Bytes for an actual counter

- A reset value for the counter

- A status byte that indicates whether a virtual interrupt has occurred and whether the VIRQ is to be reinstalled in the table after being issued

F$VIRQ also specifies an initial tick count for the interrupt. The actual call is summarized here and is described in detail in Chapter 8.

| Call: | os9 F$VIRQ |
| --- | --- |
| Input: | (Y) = address of 5-byte packet (X) = 0 to delete entry, 1 to install entry (D) = initial count value |
| Output: | None (CC) carry set on error (IS) appropriate error code |

The 5-byte packet is defined as follows:

| Name | Offset | Function |
| --- | --- | --- |
| Vi.Cnt | $0 | Actual counter |
| Vi.Rst | $2 | Reset value for counter |
| Vi.Stat | $4 | Status byte |

Two of the bits in the status byte are used. These are:

Bit 0 – set if a VIRQ occurs

Bit 7 – set if a count reset is required

When making an F$VIRQ call, the packet might require initialization with a reset value. Bit 7 of the status byte must be either set or cleared to signify a reset of the counter or a one-time VIRQ call. The reset value does not need to be the same as the initial counter value. When NitrOS-9 processes the call, it writes the packet address into the VIRQ table.

At each clock tick, NitrOS-9 scans the VIRQ table and subtracts one from each timer value. When a timer count reaches zero, NitrOS-9 performs the following actions:

- Sets bit 0 in the status byte. This specifies a Virtual IRQ.

- Checks bit 7 of the status byte for a count reset request.

- If bit 7 is set, resets the count using the reset value. If bit 7 is reset, deletes the packet address from the VIRQ table.

When a counter reaches zero and makes a virtual interrupt request, NitrOS-9 runs the standard interrupt polling routine and services the interrupt. Because of this, you must install entries on both the VIRQ and IRQ polling tables whenever you are using a VIRQ.

Unless the device has an actual physical interrupt, install the device on the IRQ polling table via the F$IRQ system call before placing it on the VIRQ table.

If the device has a physical interrupt, use the interrupt’s hardware register address as the polling address for the F$IRQ call. After setting the polling address, set the flip and mask bytes for the device and make the F$IRQ call.

If the device is totally VIRQ-driven, and has no interrupts, use the status byte from the VIRQ packet as the status byte. Use a mask byte of %00000001, defined as Vi.IFlag in the os9defs file. Use a flip byte value of 0.

See the appendix for example code using the VIRQ feature of NitrOS-9.

# Memory Modules

In Chapter 2, you learned that NitrOS-9 is based on the concept that memory is modular. This means that each program is considered to be an individually named object.

You also learned that each program loaded into memory must be in the module format. This format lets NitrOS-9 manage the logical contents of memory, as well as the physical contents. Module types and formats are discussed in detail in this chapter.

## Module Types

There are several types of modules. Each has a different use and function. These are the main requirements of a module:

- It cannot modify itself.

- It must be position-independent so that NitrOS-9 can load or relocate it wherever space is available. In this respect, the module format is the NitrOS-9 equivalent of load records used in older operating systems.

A module need not be a complete program or even 6809 machine language. It can contain BASIC09 I-code, constants, single subroutines, and subroutine packages.

## Module Format

Each module has three parts: a module header, a module body, and a cyclic-redundancy-check value (CRC value).

Module Header

Program

Or

Constants

CRC Value

### Module Header

At the beginning of the module (the lowest address) is the module header. Its form depends upon the module’s use. The header contains information about the module and its use. This information includes the following:

- Size

- Type (machine code, BASIC09 compiled code, and so on)

- Attributes (executable, re-entrant, and so on)

- Data storage memory requirements

- Execution starting address

Usually, you do not need to write routines to generate the modules and headers. All OS-9 programming languages automatically create modules and headers.

### Module Body

The module body contains the program or constants. It usually is pure code. The module name string is included in this area.

The following figure provides the offset values for calculating the location of a module’s name. (See “Offset to Module Name.”)

### CRC Value

The last three bytes of the module are the Cyclic Redundancy Check (CRC) value. The CRC value is used to verify the integrity of a module.

When the system first loads the module into memory, it performs a 24-bit CRC over the entire module, from the first byte of the module header to the byte immediately before the CRC. The CRC polynomial used is $800FE3.

As with the header, you usually don’t need to write routines to generate the CRC value. Most OS-9 programs do this automatically.

## Module Headers: Standard Information

The first nine bytes of all module headers are defined as follows:

| Relative Address | Use |
| --- | --- |
| $00,$01 | Sync bytes ($87,$CD) |
| $02,$03 | Module size |
| $04,$05 | Offset to module name |
| $06 | Module type/language |
| $07 | Attributes/revision level |
| $08 | Header check |

### Sync Bytes

The sync bytes specify the location of the module. (The first sync byte is the start of the module.) These two bytes are constant.

### Module Size

The module size specifies the size of the module in bytes (includes CRC).

### Offset to Module Name

The offset to module name specifies the address of the module name string relative to the start of the module. The name string can be located anywhere in the module. It consists of a string of ASCII characters with the most significant bit set on the last character.

### Type/Language Byte

The type/language byte specifies the type and language of the module.

The four most significant bits of this byte indicate the type. Eight types are predefined. Some of these are for OS-9’s internal use only. The type codes are given here (0 is not a legal type code):

| Code | Module Type | Name |
| --- | --- | --- |
| $1x | Program module | Prgrm |
| $2x | Subroutine module | Sbrtn |
| $3x | Multi-module (for future use) | Multi |
| $4x | Data module | Data |
| $5x-$Bx | User-definable module |  |
| $Cx | NitrOS-9 system module | Systm |
| $Dx | NitrOS-9 file manager module | FlMgr |
| $Ex | NitrOS-9 device driver module | Drivr |
| $Fx | NitrOS-9 device descriptor module | Devic |

The four least significant bits of Byte 6 indicate the language (denoted by x in the previous Figure). The language codes are given here:

| Code | Language |
| --- | --- |
| $x0 | Data (non executable) |
| $x1 | 6809 object code |
| $x2 | Basic09 I-Code |
| $x3 | Pascal P-Code |
| $x4-$xF | Reserved for future use |

By checking the language type, high-level language runtime systems can verify that a module is the correct type before attempting execution. Basic09, for example, can run either I-Code or 6809 machine language procedures arbitrarily by checking the language type code.

### Attributes/Revision Level Byte

The attributes/revision level byte defines the attributes and revision level of the module.

The four most significant bits of this byte are reserved for module attributes. Currently, only Bit 7 is defined. When set, it indicates the module is re-entrant and, therefore, shareable.

The four least significant bits of this byte are a revision level in the range 0 to 15. If two or more modules have the same name, type, language, and so on, NitrOS-9 keeps in the module directory only the module having the highest revision level. Therefore, you can replace or patch a ROM module, simply by loading a new, equivalent module that has a higher revision level.

Note: A previously linked module cannot be replaced until its link count goes to zero.

### Header Check

The header check byte contains the one’s complement of the Exclusive-OR of the previous eight bytes.

## Module Headers: Type-Dependent Information

More information usually follows the first nine bytes of a module header. The layout and meaning vary, depending on the module type.

Module types $Cx-$Fx (system module, file manager module, device driver module, and device descriptor module) are used only by OS-9. Their formats are given later in the manual.

Module types $lx through $Bx have a general-purpose executable format. This format is often used in programs called by F$Fork or F$Chain. Here is the format used by these module types:

Sync Bytes ($87,$CD)

Module Size (bytes)

Module Name Offset

Type

Language

Attributes

Revision

Header Parity Check

Execution Offset

Permanent Storage Size

(Additional optional header

extensions)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

Module Body, object code, constants, and so on

24-bit CRC Check Value

Module

CRC

Header

parity

$00

$01

$02

$03

$04

$05

$06

$07

$08

$09

$0A

$0B

$0C

$0D

As you can see from the preceding chart, the executable memory has four extra bytes in its header. They are:

$09,$0A		Execution offset

$0B,$0C		Permanent storage size

Execution Offset. The program or subroutine’s offset starting address, relative to the first byte of the sync code. A module that has multiple entry points (such as cold start and warm start) might have a branch table starting at this address.

Permanent Storage Size. The minimum number of bytes of data storage required to run. Fork and Chain use this number to allocate a process’ data area.

If the module is not directly executed by a Fork or Chain system call (for instance a subroutine package), this entry is not used by NitrOS-9. It is commonly used to specify the maximum stack size required by re-entrant subroutine modules. The calling program can check this value to determine if the subroutine has enough stack space.

When NitrOS-9 starts after a single system reset, it searches the entire memory space for ROM modules. It finds them by looking for the module header sync code ($87,$CD).

When NitrOS-9 detects the header sync code, it checks to see if the header is correct. If it is, the system obtains the module size from the header and performs a 24-bit CRC over the entire module. If the CRC matches, NitrOS-9 considers the module to be valid and enters it into the module directory. All ROM modules that are present in the system at startup are automatically included in the system module directory.

After the module search, NitrOS-9 links to the component modules it found. This is the secret to NitrOS-9’s ability to adapt to almost any 6809 computer. It automatically locates its required and optional component modules and rebuilds the system each time it is started.

# NitrOS-9’s Unified Input/Output System

Chapter 1 mentioned that NitrOS-9 has a unified I/O system, consisting of all modules except those at the kernel level. This chapter discusses the I/O modules in detail.

The VDG interface performs both interface and low-level routines for VDG Color Computer 2 compatible modes and has limited support for high resolution screen allocation.

The GrfInt interface provides the standard code interpretations and interface functions.

The WindInt interface, available in the Multi-View package, contains all the functionality of GrfInt along with additional support features. If you use WindInt, do not include Grflnt.

Both WindInt and Grflnt use the low-level driver Grfdrv to perform drawing on the bitmap screens.

Term_VDG uses CC3I0 VdgInt while Term_win and all window descriptors use CC3I0/(WindInt/Grflnt)/GrfDrv modules.

The I/O system provides system-wide, hardware-independent I/O services for user programs and OS-9 itself. All I/O system calls are received by the kernel and passed to the I/O manager for processing.

The I/O manager performs some processing, such as the allocation of data structures for the I/O path. Then, it calls the file managers and device drivers to do most of the work. Additional file manager, device driver, and device descriptor modules can be loaded into memory from files and used while the system is running.

## The I/O Manager

The I/O manager provides the first level of service of I/O system calls. It routes data on I/O process paths to and from the appropriate file managers and device drivers.

The I/O Manager also maintains two important internal OS-9 data structures: the device table and the path table. Never modify the I/O manager.

When a path is opened, the I/O manager tries to link to a memory module that has the device name given or implied in the pathlist. This module is the device descriptor. It contains the names of the device driver and file manager for the device. The I/O manager saves the names so later system calls can be routed to these modules.

## File Managers

NitrOS-9 can have any number of file manager modules. Each of these modules processes the raw data stream to or from a class of device drivers that have similar operational characteristics. It removes as many unique characteristics as possible from I/O operations. Thus, it assures that similar devices conform to the NitrOS-9 standard I/O and file structure.

The file manager also is responsible for mass storage allocation and directory processing, if these are applicable to the class of devices it serves. File managers usually buffer the data stream and issue requests to the kernel for dynamic allocation of buffer memory. They can also monitor and process the data stream, for example, adding linefeed characters after carriage-return characters.

The file managers are re-entrant. The three standard NitrOS-9 file managers are:

- Random block file manager: The RBF manager supports random-access, block-structured devices such as disk systems and bubble memories. (Chapter 5 discusses the RBF manager in detail.)

- Sequential Character File Manager: The SCF manager supports single-character-oriented devices, such as CRTs or hardcopy terminals, printers, and modems. (Chapter 6 discusses SCF in detail.)

- Pipe File Manager (PIPEMAN): The pipe manager supports interprocess communication via pipes.

### File Manager Structure

Every file manager must have a branch table in exactly the following format. Routines that are not used by the file manager must branch to an error routine that sets the carry and loads B with an appropriate error code before returning. Routines returning without error must ensure that the carry bit is clear.

* All routines are entered with:

* (Y) = Path Descriptor pointer

* (U) = Caller’s register stack pointer

*

EntryPt  equ		*

lbra 	Create

lbra 	Open

lbra 	MakDir

lbra 	ChgDir

lbra 	Delete

lbra 	Seek

lbra 	Read

lbra 	Write

lbra 	ReadLn

lbra 	WriteLn

lbra 	GetStat

lbra 	PutStat

lbra 	Close

### Create, Open

Create and Open handle file creating and opening for devices. Typically, the process involves allocating any required buffers, initializing path descriptor variables, and establishing the path name. If the file manager controls multi-file devices (RBF), directory searching is performed to find or create the specified file.

### MakDir

MakDir creates a directory file on multi-file devices. MakDir is neither preceded by a Create nor followed by a Close. File managers that are incapable of supporting directories need to return carry set with an appropriate error code in Register B.

### ChgDir

On multi-file devices, ChgDir searches for a directory file. If ChgDir finds the directory, it saves the address of the directory (up to four bytes) in the caller’s process descriptor. The descriptor is located at P$DIO + 2 (for a data directory) or P$DIO + 8 (for an execution directory).

In the case of the RBF manager, the address of the directory’s file descriptor is saved. Open/Create begins searching in the current directory when the caller’s pathlist does not begin with a slash. File managers that do not support directories should return the carry set and an appropriate error code in Register B.

### Delete

Multi-file device managers handle file delete requests by initiating a directory search that is similar to Open. Once a device manager finds the file, it removes the file from the directory.

Any media in use by the file are returned to unused status. In the case of the RBF manager, space is returned for system use and is marked as available in the free cluster bitmap on the disk. File managers that do not support multi-file devices return an error.

### Seek

File managers that support random access devices use Seek to position file pointers of an already open path to the byte specified. Typically, the positioning is a logical movement. No error is produced at the time of the seek if the position is beyond the current “end of file.”

Normally, file managers that do not support random access ignore Seek, However, an SCF-type manager can use Seek to perform cursor positioning.

### Read

Read returns the number of bytes requested to the user’s data buffer. Make sure Read returns an EOF error if there is no data available. Read must be capable of copying pure binary data, and generally performs no editing on the data. Generally, the file manager calls the device driver to actually read the data into the buffer. Then, the file manager copies the data from the buffer into the user’s data area to keep file managers device independent.

### Write

The Write request, like Read, must be capable of recording pure binary data without alteration. The routines for Read and Write are almost identical with the exception that Write uses the device driver’s output routine instead of the input routine. The RBF manager and similar random access devices that use fixed length records (sectors) must often pre-read a sector before writing it, unless they are writing the entire sector. In OS-9, writing past the end of file on a device expands the file with new data.

### ReadLn

ReadLn differs from Read in two respects. First, ReadLn terminates when the first end-of-line (carriage return) is encountered. ReadLn performs any input editing that is appropriate for the device. In the case of SCF, editing involves handling functions such as backspace, line deletion, and the removal of the high order bit from characters.

### WriteLn

WriteLn is the counterpart of ReadLn. It calls the device driver to transfer data up to and including the first (if any) carriage return encountered. Appropriate output editing can also be performed. For example, SCF outputs a line feed, a carriage return character, and nulls (if appropriate for the device). It also pauses at the end of a screen page.

### GetStat, PutStat

The GetStat (get status) and PutStat (put status) system calls are wildcard calls designed to provide a method of accessing features of a device (or file manager) that are not generally device independent. The file manager can perform specific functions such as setting the size of a file to a given value. Pass unknown status calls to the driver to provide further means of device independence. For example, a PutStat call to format a disk track might behave differently on different types of disk controllers.

### Close

Close is responsible for ensuring that any output to a device is completed. (If necessary, Close writes out the last buffer.) It releases any buffer space allocated in an Open or Create. Close does not execute the device driver’s terminate routine, but can do specific end-of-file processing if you want it to, such as writing end-of-file records on disks, or form feeds on printers.

## Interfacing with Device Drivers

Strictly speaking, device drivers must conform to the general format presented in this manual. The I/O Manager is slightly different because it only uses the Init and Terminate entry points.

Other entry points need only be compatible with the file manager for which the driver is written. For example, the Read entry point of an SCF driver is expected to return one byte from the device. The Read entry point of an RBF driver, on the other hand, expects Read to return an entire sector.

The following code is part of an SCF file manager. The code shows how a file manager might call a driver.

********************

* IOEXEC

*	Execute Device's Read/Write Routine

*

* Passed:	(A) = Output character (write)

*			(X) = Device Table entry ptr

*			(Y) = Path Descriptor pointer

*			(U) = Offset of routine (D$Read,D$Write)

*

* Returns:	(A) = Input char (read)

*			(B) = Error code, CC set if error

*

* Destroys B,CC

IOEXEC	pshs	 a,x,y,u save registers

ldu	V$STAT,x get static storage for driver

ldx	V$DRIV,x get driver module address

ldd 	M$EXEC,x and offset of execution entries

addd 5,s offset by read/write

leax d,x absolute entry address

lda ,s+ restore char (for write)

jsr ,x execute driver read/write

puls x,y,u,pc return (A)=char, (B)=error

emod 	Module CRC

Size 	equ 	* 	size of sequential file manager

## Device Driver Modules

The device driver modules are subroutine packages that perform basic, low-level I/O transfers to or from a specific type of I/O device hardware controller. These modules are re-entrant. So, one copy of the module can concurrently run several devices that use identical I/O controllers.

Device driver modules use a standard module header, in which the module type is specified as code $Ex (device driver). The execution offset address in the module header points to a branch table that has a minimum of six 3-byte entries.

Each entry is typically an LBRA to the corresponding subroutine. The file managers call specific routines in the device driver through this table, passing a pointer to a path descriptor and passing the hardware control register address in the 6809 registers. The branch table looks like this:

| Code | Meaning |
| --- | --- |
| $00 | Device initialization routine |
| $03 | Read form device |
| $06 | Write to device |
| $09 | Get device status |
| $0C | Set device status |
| $0F | Device termination routine |

(For a complete description of the parameters passed to these subroutines, see the “Device Driver Subroutines” sections in Chapters 5 and 6.)

Sync Bytes ($87,$CD)

Module Size (bytes)

Header

parity

Module Name Offset

Type

Language

Attributes

Revision

Header Parity Check

Module

CRC

Execution Offset

Permanent Storage Size

Mode Byte

Module Body

24-bit CRC Check Value

## NitrOS-9 Interaction with Devices

Device drivers often must wait for hardware to complete a task or for a user to enter data. Such a wait situation occurs if an SCF device driver receives a Read but there is no data is available, or if it receives a Write and no buffer space is available. NitrOS-9 drivers that encounter this situation should suspend the current process (via F$Sleep). In this way the driver allows other processes to continue using CPU time.

The most efficient way for a driver to awaken itself and resume processing data is by using interrupt requests (IRQs). It is possible for the driver to sleep for a number of system clock ticks and then check the device or buffer for a ready signal. The drawbacks to this technique are:

- It requires the system clock to always remain active.

- It might require a large number of ticks (perhaps 20) for the device to become ready. Such a case leaves you with a dilemma. If you make the program sleep for two ticks, the system wastes CPU time while checking for device ready. If the driver sleeps 20 ticks, it does not have a good response time.

An interrupt system allows the hardware to report to the CPU and the device drivers when the device is finished with an operation. Using interrupts to its advantage, a device driver can setup interrupt handling to occur when a character is sent or received or when a disk operation is complete. There is a built-in polling facility for pausing and awakening processes. Here is a technique for handling interrupts in a device driver:

- Use the Init routine to place the driver interrupt service call (IRQSVC) routine in the IRQ polling sequence via an F$IRQ system call:

ldd V.Port,u get address to poll

leax IRQPOLL,pcr point to IRQ packet

leay IRQSERVC,pcr point to IRQ routine

os9 F$IRQ add dev to poll Sequence

bcs Error abnormal exit if error

- Ensure that driver programs waiting for their hardware call the sleep routine. The sleep routine copies V.Busy to V.Wake. Then, it goes to sleep for some period of time.

- When the driver program wakes up, have it check to see whether it was awakened by an interrupt or by a signal sent from some other process.

Usually, the driver performs this check by reading the V.Wake storage byte. The V.Busy byte is maintained by the file manager to be used as the process ID of the process using the driver. When V.Busy is copied into V.Wake, then V.Wake becomes a flag byte and an information byte. A non-zero Wake byte indicates that there is a process awaiting an interrupt. The value in the Wake byte indicates the process to be awakened by sending a wakeup signal as shown in the following code:

lda V.Busy,u get proc ID

sta V.Wake,u arrange for wakeup

andcc #^IntMasks prep for interrupts

Sleep50 ldx #0 or any other tick time (if signal test )

OS9 F$Sleep await an IRQ

ldx D.Proc get proc desc ptr if signal test

ldb P$Signal,x i5 signal present? (if signal test)

bne SigTest bra if 50 if Signal test

tst V.Wake,u IRQ occur?

bne Sleep50 bra if not

Note that the code labeled “if signal test” is only necessary if the driver wishes to return to the caller if a signal is sent without waiting for the device to finish. Also note that IRQs and FIRQs must be masked between the time a command is given to the device and the moving of V.Busy and V .Wake. If they are not masked, it is possible for the device IRQ to occur and the IRQSERVC routine to become confused as to whether it is sending a wakeup signal or not.

- When the device issues an interrupt, NitrOS-9 calls the routine at the address given in F$IRQ with the interrupts masked. Make the routine as short as possible, and have it return with an RTS instruction. IRQSERVC can verify that an interrupt has occurred for the device. It needs to clear the interrupt to retrieve any data in the device. Then the V.Wake byte communicates with the main driver module. If V.Wake is non-zero, clear it to indicate a true device interrupt and use its contents as the process ID for an F$Send system call. The F$Send call sends a wakeup signal to the process. Here is an example:

ldx V.Port,u get device address

tst ?? is it real interrupt from device?

bne IRQSVC90 bra to error if not

lda Data,x get data from device

sta 0,y

lda V.Wake,u

beq IRQSVC80 bra if none

clr V.Wake,u clear it as flag to main routine

ldb #S$Wake,u get wakeup signal

os9 F$Send Send Signal to driver

IRQSVC80 clrb clear carry bit (all is well)

rts

IROSVC90 comb Set carry bit (is an IRQ call)

rts

### Suspend State (NitrOS-9 Level 2 only)

The Suspend State allows the elimination of the F$Send system call during interrupt handling. Because the process is already in the active queue, it need not be moved from one queue to another. The device driver IRQSERVC routine can now wake up the suspended main driver by clearing the process status byte suspend bit in the process state. Following are sample routines for the Sleep and IRQSERVC calls:

lda D.Proc get process ptr

sta V.Wake,u prep for re-awakening

enable device to IRQ, give command, etc.

bra Cmd5O enter suspend loop

Cmd30 ldx D.Proc get ptr to process desc

lda P$State,x get state flag

ora Suspend put proc in suspend state

sta P$State,x save it in proc desc

andcc #^IntMasks unmask interrupts

ldx #1 give up time slice

OS9 F$Sleep suspend (in active queue)

Cmd50 orcc #IntMasks mask interrupts while changing state

ldx D.Proc get proc desc addr (if signal test)

lda P$Signal,x get signal (if signal test)

beq SigProc bra if signal to be handled

lda V.Wake,u true interrupt?

bne Cmd30 bra if not

andcc #^IntMasks assure interrupts unmasked

Note that D.Proc is a pointer to the process descriptor of the current process. Process descriptors are always allocated on 256 byte page boundaries. Thus, having the high order byte of the address is adequate to locate the descriptor. D.Proc is put in V.Wake as a dual value. In one instance, it is a flag byte indicating that a process is indeed suspended. In the other instance, it is a pointer to the process descriptor which enables the IRQSERVC routine to clear the suspend bit. It is necessary to have the interrupts masked from the time the device is enabled until the suspend bit has been set. Making the interrupts ensure that the IRQSERVC routine does not think it has cleared the suspend bit before it is even set. If this happens, when the bit is set the process might go into permanent suspension. The IRQSERVC routine sample follows:

ldy V.Port,u get dev addr

tst V.Wake,u is process awaiting IRQ?

beq IRQSVCER no exit

clear device interrupt

exit if IRQ not from this device

lda V.Wake,u get process ptr

clrb

stb V.Wake,u clear proc waiting flag

tfr d,x get process descriptor ptr

lda P$State,x get state flag

anda # Suspend clear suspend state

sta P$State,x save it

clrb clear carry bit

rts

IRQSVCER comb Set carry bit

rts

### Device Descriptor Modules

Device descriptor modules are small, non-executable modules. Each one provides information that associates a specific I/O device with its logical name, hardware controller address(es), device driver, file manager name, and initialization parameters.

Unlike the device drivers and file managers, which operate on classes of devices, each device descriptor tailors its functions to a specific device. Each device must have a device descriptor.

Device descriptor modules use a standard module header, in which the module type is specified as code $Fx (device descriptor). The name of the module is the name by which the system and user know the device (the device name given in path lists).

The rest of the device descriptor header consists of the information in the following chart:

| Relative Address(es) | Use |
| --- | --- |
| $09,$OA | The relative address of the file manager name string address |
| $OB,$OC | The relative address of the device driver name string |
| $OD | Mode/Capabilities: D S PE PW PR E W R (directory, single user, public execute, public write, public read, execute, write, read) |
| $OE,$OF,$10 | The absolute physical (24-bit) address of the device controller |
| $11 | The number of bytes (n bytes) in the initialization table |
| $12,$12 + n | Initialization table |

When OS-9 opens a path to the device, the system copies the initialization table into the option section (PD.OPT) of the path descriptor. (See “Path Descriptors” in this chapter.)

The values in this table can be used to define the operating parameters that are alterable by the Get Status and Set Status system calls (I$GetStt and I$SetStt). For example, parameters that are used when initializing terminals define which control characters are to be used for functions such as backspace and delete.

The initialization table can be a maximum of 32 bytes long. If the table is fewer than 32 bytes long, OS-9 sets the remaining values in the path descriptor to 0.

You might wish to add devices to your system. If a similar device driver already exists, all you need to do is add the new hardware and load another device descriptor. Device descriptors can be in the boot module or they can be loaded into RAM from mass-storage files while the system is running.

The following diagram illustrates the device descriptor format:

Device Descriptor Format

| Name | Relative Address | Bytes | Use |
| --- | --- | --- | --- |
| M$ID | $00-$01 | 2 | Sync Bytes ($87CD) |
| M$Size | $02-$03 | 2 | Module Size (bytes) |
| M$Name | $04-$05 | 2 | Offset to Module Name |
| M$Type | $06 | 1 | Type / Language |
| M$Revs | $07 | 1 | Attributes / Revision Level |
| M$Parity | $08 | 1 | Header Parity Check |
| M$FMgr | $09-$0A | 2 | File Manager Name Offset |
| M$PDev | $0B-$0C | 2 | Device Driver Name Offset |
| M$Mode | $0D | 1 | Mode |
| M$Port | $0E-$10 | 3 | Port Address |
| M$Opt | $11 | 1 | Initialization Table Size |
|  | $12,$12…n | n | Initialization table |
|  |  |  | Name Strings, and so on |
|  |  |  | CRC Check Value |

## Path Descriptors

Every open path is represented by a data structure called a path descriptor (PD). The PD contains the information the file managers and device drivers require to perform I/O functions.

PDs are 64 bytes long and are dynamically allocated and deallo cated by the I/O manager as paths are opened and closed.

They are internal data structures that are not normally referenced from user or applications programs. The description of PDs is presented here mainly for those programmers who need to write custom file managers, device drivers, or other extensions to OS-9.

PDs have three sections. The first section, which is ten bytes long, is the same for all file managers and device drivers. The information in the first section is shown in the following chart.

Path Descriptor: Standard Information

| Name | Address | Bytes | Use |
| --- | --- | --- | --- |
| PD.PD | $00 | 1 | Path number |
| PD.MOD | $01 | 1 | Access mode: 1 = read, 2 = write, 3 = update |
| PD.CNT | $02 | 1 | Number of open paths using this PD |
| PD.DEV | $03 | 2 | Address of the associated device table entry |
| PD.CPR | $05 | 1 | Current process ID |
| PD.RGS | $06 | 2 | Address of the caller’s register stack |
| PD.BUF | $08 | 2 | Address of the 256-byte data buffer (if used) |
| PD.FST | $OA | 22 | Defined by the file manager |
| PD.OPT | $20 | 32 | Reserved for the GetStat/SetStat options |

PD.FST is 12-byte storage reserved for and defined by each type of file manager for file pointers, permanent variables, and so on.

PD.OPT is a 32-byte option area used for file or device operating parameters that are dynamically alterable. When the path is opened, the I/O manager initializes these variables by copying the initialization table that is in the device descriptor module. User programs can change the values later, using the Get Status and Set Status system calls.

PD.FST and PD.OPT are defined for the file manager in the assembly-language equate file (SCFDefs for the SCF manager or RBFDefs for the RBF manager).

# Random Block File Manager

The random block file manager (RBF manager) supports disk storage. It is a re-entrant subroutine package called by the I/O manager for I/O system calls to random-access devices. It maintains the logical and physical file structures.

During normal operation, the RBF manager requests allocation and deallocation of 256-byte data buffers. Usually, one buffer is required for each open file. When physical I/O functions are necessary, the RBF manager directly calls the subroutines in the associated device drivers. All data transfers are performed using 256-byte data blocks (pages).

The RBF manager does not deal directly with physical addresses such as tracks and cylinders. Instead, it passes to the device drivers address parameters, using a standard address called a logical sector number, or LSN. LSNs are integers from 0 to n-1, where n is the maximum number of sectors on the media. The driver translates the logical sector number to actual cylinder/track/sector values.

Because the RBF manager supports many devices that have different performance and storage capacities, it is highly parameter-driven. They physical parameters it uses are stored on the media itself.

On disk systems, the parameters are written on the first few sectors of Track 0. The device drivers also use the information, particularly the physical parameters stored on Sector 0. These parameters are written by the FORMAT program that initializes and tests the disk.

## Logical and Physical Disk Organization

All disks used by NitrOS-9 store basic information, file structure, and storage allocation information on these first few sectors.

LSN 0 is the identification sector. LSN 1 is the disk allocation map sector. LSN 2 marks the beginning of the disk’s root directory. The following section tells more about LSN 0 and LSN 1.

### Identification Sector (LSN 0)

LSN 0 contains a description of the physical and logical characteristics of the disk. These characteristics are set by the FORMAT command program when the disk is initialized.

The following table gives the NitrOS-9 mnemonic name, byte address, size, and description of each value stored in this LSN 0.

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| DD.TOT | $00 | 3 | Number of sectors on disk |
| DD.TKS | $03 | 1 | Track size (in sectors) |
| DD.MAP | $04 | 2 | Number of bytes in the allocation bit map |
| DD.BIT | $06 | 2 | Number of sectors per cluster |
| DD.DIR | $08 | 3 | Starting sector of the root directory |
| DD.OWN | $0B | 2 | Owner’s user number |
| DD.ATT | $0D | 1 | Disk attributes |
| DD.DSK | $0E | 2 | Disk identification (for internal use) |
| DD.FMT | $10 | 1 | Disk format, density, number of sides |
| DD.SPT | $11 | 2 | Number of sectors per track |
| DD.RES | $13 | 2 | Reserved for future use |
| DD.BT | $15 | 3 | Starting sector of the bootstrap file |
| DD.BSZ | $18 | 2 | Size of the bootstrap file (in bytes) |
| DD.DAT | $1A | 5 | Time of creation (Y:M:D:H:M) |
| DD.NAM | $1F | 32 | Volume name in which the last character has the most significant bit set |
| DD.OPT | $3F |  | Path descriptor options |

### Disk Allocation Map Sector (LSN 1)

LSN 1 contains the disk allocation map, which is created by FORMAT. This map shows which sectors are allocated to the files and which are free for future use.

Each bit in the allocation map represents a sector or cluster of sectors on the disk. If the bit is set, the sector is considered to be in use, defective, or non-existent. If the bit is cleared, the corresponding cluster is available. The allocation map usually starts at LSN 1. The number of sectors it requires varies according to how many bits are needed for the map. DD.MAP specifies the actual number of bytes used in the map.

Multiple sector allocation maps allow the number of sectors/cluster to be as small as possible for high volume media.

The FORMAT utility bases the size of the allocation map on the size and number of sectors per cluster.

The DD.MAP value in LSN 0 specifies the number of bytes (in LSN 1) that are used in the map.

Each bit in the disk allocation map corresponds to one sector cluster on the disk. The DD.BIT value in LSN 0 specifies the number of sectors per cluster. The number is an integral power of 2 (1, 2, 4, 8, 16, and so on).

If a cluster is available, the corresponding bit is cleared. If it is allocated, non-existent, or physically defective, the corresponding bit is set.

### Root Directory

The file is the parent directory of all other files and directories on the disk. It is the directory accessed using the physical device name (such as /D1). Usually, it immediately follows the Allocation Map. The location of the root directory file descriptor is specified in DD.DIR. The root directory contains an entry for each file that resides in the directory, including other directories.

### File Descriptor Sector

The first sector of every file is the file descriptor. It contains the logical and physical description of the file.

The following table describes the contents of the file descriptor.

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| FD.ATT | $00 | 1 | File attributes: D S PE PW PR E W R (see next chart) |
| FD.OWN | $01 | 2 | Owner’s user ID |
| FD.DAT | $03 | 5 | Date last modified (Y M D H M) |
| FD.LNK | $08 | 1 | Link count |
| FD.SIZ | $09 | 4 | File size (number of bytes) |
| FD.CREAT | $0D | 3 | Date created (Y M D) |
| FD.SEG | $10 | 240 | Segment list (see next chart) |

FD.ATT. The attribute byte contains the file permission bits. When set the bits indicate the following

Bit 7	Directory

Bit 6	Single user

Bit 5	Public execute

Bit 4	Public write

Bit 3	Public read

Bit 2	Execute

Bit 1	Write

Bit 0	Read

FD.SEG. The segment list consists of a maximum of 48 5-byte entries that have the size and address of each file block in logical order. Each entry has the block’s 3-byte LSN and 2-byte size (in sectors). The entry following the last segment is zero.

After creation, a file has no data segments allocated to it until the first write. (Write operations past the current end-of-file cause sectors to be added to the file. The first write is always past the end-of-file.)

If the file has no segments, it is given an initial segment. Usually, this segment has the number of sectors specified by the minimum allocation entry in the device descriptor. If, however, the number of sectors requested is more than the minimum, the initial segment has the requested number.

Later expansions of the file usually are also made in minimum allocation increments. Whenever possible, NitrOS-9 expands the last segment instead of adding a segment. When the file is closed, NitrOS-9 truncates unused sectors in the last segment.

NitrOS-9 tries to minimize the number of storage segments used in a file. In fact, many files have only one segment. In such cases, no extra read operations are needed to randomly access any byte in the file.

If a file is repeatedly closed, opened, and expanded, it can become fragmented so that it has many segments. You can avoid this fragmentation by writing a byte at the highest address you want to be used on a file. Do this before writing any other data.

## Directory

Disk directories are files that have the D attribute set. A directory contains an integral number of entries, each of which can hold the name and LSN of a file or another directory.

Each directory entry contains 29 bytes for the filename followed by three bytes for the LSN of the file’s descriptor sector. The filename is left-justified in the field with the most significant bit of the last character set. Unused entries have a zero byte in the first filename character position.

Every disk has a master directory called the root directory. The DD.DIR value in LSN 0 (identification sector) specifies the starting sector of the root directory.

## The RBF Manager Definitions of the Path Descriptor

As stated earlier in this chapter, the PD.FST section of the path descriptor is reserved for and defined by the file manager. The following table describes the use of this section by the RBF manager. For your convenience, it also includes the other sections of the PD.

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| Universal Section (Same for all file managers and device drivers) |  |  |  |
| PD.PD | $00 | 1 | Path number |
| PD.MOD | $01 | 1 | Access mode 1 = read 2 = write 3 = update |
| PD.CNT | $02 | 1 | Number of open images (paths using this PD) |
| PD.DEV | $03 | 2 | Address of the associated device table entry |
| PD.CPR | $05 | 1 | Current process ID |
| PD.RGS | $06 | 2 | Address of the caller’s 6809 register stack |
| PD.BUF | $08 | 2 | Address of the 256-byte data buffer (if used) |

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| The RBF manager Path Descriptor Definitions (PD.FST Section) |  |  |  |
| PD.SMF | $0A | 1 | State flag: Bit 0 = current buffer is altered Bit 1 = current sector is in the buffer Bit 2 = descriptor sector is in the buffer |
| PD.CP | $0B | 4 | Current logical file position (byte address) |
| PD.SIZ | $0F | 4 | File size |
| PD.SBL | $13 | 3 | Segment beginning logical sector number (LSN) |
| PD.SBP | $16 | 3 | Segment beginning physical sector number (PSN) |
| PD.SSZ | $19 | 3 | Segment size |
| PD.DSK | $1C | 2 | Disk ID (for internal use only) |
| PD.DTB | $1E | 2 | Address of drive table |

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| The RBF manager Option Section Definitions (PD.OPT Section) (Copied from the device descriptor) |  |  |  |
| PD.DTP | $20 | 1 | Device class 0 = SCF 1 = RBF 2 = PIPE 3 = SBF |
| PD.DRV | $21 | 1 | Drive number (0..n) |
| PD.STP | $22 | 1 | Step rate |
| PD.TYP | $23 | 1 | Device type |
| PD.DNS | $24 | 1 | Density capability |
| PD.CYL | $25 | 2 | Number of cylinders (tracks) |
| PD.SID | $27 | 1 | Number of sides (surfaces) |
| PD.VFY | $28 | 1 | 0 = verify disk writes |
| PD.SCT | $29 | 2 | Default number of sectors per track |
| PD.T0S | $2B | 2 | Default number of sectors per track (Track 0) |
| PD.ILV | $2D | 1 | Sector interleave factor |
| PD.SAS | $2E | 1 | Segment allocation size |
| PD.TFM | $2F | 1 | DMA transfer mode |
| PD.EXTEN | $30 | 2 | Path extension for record locking |
| PD.STOFF | $32 | 1 | Sector/track offsets |
| (Not copied from the device descriptor) |  |  |  |
| PD.ATT | $33 | 1 | File attributes (D S PE PW PR E W R) |
| PD.FD | $34 | 3 | File descriptor PSN |
| PD.DFD | $37 | 3 | Directory file descriptor PSN |
| PD.DCP | $3A | 4 | File’s directory entry pointer |
| PD.DVT | $3E | 2 | Address of the device table entry |

Any values not determined by this table default to zero.

## RBF-Type Device Descriptor Modules

This section describes the use of the initialization table contained in the device descriptor modules for RBF-type devices. The following values are those the I/O manager copies from the device descriptor to the path descriptor.

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
|  | $00-$11 |  | Standard device descriptor module header |
| IT.DTP | $12 | 1 | Device type: 0 = SCF 1 = RBF 2 = PIPE 3 = SBF |
| IT.DRV | $13 | 1 | Drive number |
| IT.STP | $14 | 1 | Step rate |
| IT.TYP | $15 | 1 | Device type (see RBF path descriptor) |
| IT.DNS | $16 | 1 | Media density: Always 1 (double) (see following information) |
| IT.CYL | $17 | 2 | Number of cylinders (tracks) |
| IT.SID | $19 | 1 | Number of sides |
| IT.VFY | $1A | 1 | 0 = Verify disk writes 1 = no verify |
| IT.SCT | $1B | 2 | Default number of sectors per track |
| IT.T0S | $1D | 2 | Default number of sectors per track (Track 0) |
| IT.ILV | $1F | 1 | Sector interleave factor |
| IT.SAS | $20 | 1 | Minimum size of segment allocation (number of sectors to be allocated at one time) |

IT.DRV is used to associate a 1-byte integer with each drive that a controller handles. Number the drives for each controller as 0 to n-1, where n is the maximum number of drives the controller can handle.

IT.TYP specifies the device type (all types).

Bit 0	0 = 5-inch floppy diskette

Bit 5	0 = Non-Color Computer format

1 = Color Computer format

Bit 6	0 = Standard NitrOS-9 format

1 = Non-standard format

Bit 7	0 = Floppy diskette

1 = Hard disk

IT.DNS specifies the density capabilities (floppy diskette only).

Bit 0	0 = Single-bit density (FM)

1 = Double-bit density (MFM)

Bit 1	0 = Single-track density (5-inch, 48 tracks per inch)

1 = Double-track density (5-inch, 96 tracks per inch)

IT.SAS specifies the minimum number of sectors allowed at one time.

### RBF Record Locking

Record locking is a general term that refers to methods designed to preserve the integrity of files that can be accessed by more than one user or process. The NitrOS-9 implementation of record locking is designed to be as invisible as possible. This means that existing programs do not have to be rewritten to take advantage of record locking facilities. You can usually write new programs without special concern for multi-user activity.

Record locking involves detecting and preventing conflicts during record access. Whenever a process modifies a record, the system locks out other procedures from accessing the file. It defers access to other procedures until it is safe for them to write to the record. The system does not lock records during reads; so, multiple processes can read the records at the same time.

### Record Locking and Unlocking

To detect conflicts, NitrOS-9 must recognize when a record is being updated. The RBF manager provides true record locking on a byte basis. A typical record update sequence is:

OS9 I$Read	program reads record

RECORD is LOCKED

.

.		program updates record

.

OS9 I$Seek	reposition to record

OS9 I$Write	record is rewritten

RECORD IS RELEASED

When a file is opened in update mode, any read causes locking of the record being accessed. This happens because the RBF manager cannot determine in advance if the record is to be updated. The record stays locked until the next read, write, or close.

However, when a file is opened in the read or execute modes, the system does not lock accessed records because the records cannot be updated in these two modes.

A subtle but important problem exists for programs that interrogate a data base and occasionally update its data. If you neglect to release a record after accessing it, the record might be locked indefinitely. This problem is characteristic of record locking systems and you can avoid it with careful programming.

Only one potion of a file can be locked at a time. If an application requires more than one record to be locked, open multiple paths to the same file and lock the record accessed by each path. RBF notices that the same process owns both paths and keeps them from locking each other.

### Non-Shareable Files

Sometimes (although rarely), you must create a file that can never be accessed by more than one user at a time. To lock the file, you set the single-user bit in the file’s attribute byte. You can do this by using the proper option when the file is created, or later using the NitrOS-9 ATTR command. Once the single-user bit is set, only one use can open the file at a time. If other users attempt to open the file, Error 253 is returned. Note, however, that non-shareable means only one path can be opened to a file at one time. Do not allow two processes to concurrently access a non-shareable file through the same path.

More commonly, you need to declare a file as single-user only during the execution of a specific program. You can do this by opening the file with the single-user bit set. For example, suppose a process is sorting a file. With the file’s single-user bit set, NitrOS-9 treats the file exactly as though it had a single-user attribute. If another process attempts to open the file, NitrOS-9 returns Error 253.

You can duplicate non-shareable files by using the I$Dup system call. This means that it can be inherited and therefore accessible to more than one process at a time. Single-user means only that the file can be opened only once.

### End-of-File Lock

A special case of record locking occurs when a user reads or writes data at the end of a file, creating an EOF Lock. An EOF Lock keeps the end of the file locked until a process performs a read or write that it is not at the end of the file. It prevents problems that might otherwise occur when two users want to simultaneously extend a file. The EOF Lock is the only case in which a write call automatically causes portions of a file to be locked. An interesting and useful side effect of the EOF Lock function occurs if a program creates a file for sequential output. As soon as the program creates the file, EOF Lock is set and no other process can pass the writer in processing the file. For example, if an assembler redirects a listing to a disk file, and a spooler utility tries to print a line from the file it is written, record locking makes the spooler wait and stay at least one step behind the assembler.

### Deadlock Detection

A deadly embrace, or deadlock, typically occurs when two processes attempt to gain control of two or more disk areas at the same time. If each process gets one area (locking the other process), both processes become permanently stuck. Each waits for a segment that can never become free. This situation is not restricted to any particular record locking scheme or operating system.

When a deadly embrace occurs, RBF returns a deadlock error (Error 254) to the process that caused NitrOS-9 to detect the deadlock. To avoid deadlocks, make sure that processes always access records of shared files in the same sequence.

When a deadlock error occurs, it is not sufficient for a program to retry the operation that caused the error. If all processes use this strategy, none can ever succeed. For any process to proceed, at least one must cancel operation to release control over a requesting segment.

## RBF-Type Device Driver Modules

An RBF-type device driver module contains a package of subroutines that perform sector-oriented I/O to or from a specific hardware controller. Such a module is usually re-entrant. Because of this, one copy of one device driver module can simultaneously run several devices that use identical I/O controllers.

The I/O manager allocates a permanent memory area for each device driver. The size of the memory area is given in the device driver module header. The I/O manager and the RBF manager use some of this area. The device driver can use the rest in any manner. This area is used as follows:

### The RBF Device Memory Area Definitions

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| V.PAGE | $00 | 1 | Port extended address bits A20-A16 |
| V.PORT | $01 | 2 | Device base address (defined by the I/O manager) |
| V.LPRC | $03 | 1 | ID of the last active process (not used by RBF device drivers) |
| V.BUSY | $04 | 1 | ID of the current process using driver (defined by RBF) 0 = no current process |
| V.WAKE | $05 | 1 | ID of the process waiting for I/O completion (defined by the device driver) |
| V.USER | $06 | 0 | Beginning of file manager specific storage |
| V.NDRV | $06 | 1 | Maximum number of drives the controller can use (defined by the device driver) |
|  | $07 | 8 | Reserved |
| DRVBEG | $0F | 0 | Beginning of the drive tables |
| TABLES | $0F | DRVMEM*N | Space for number of tables reserved (n) |
| FREE |  | 0 | Beginning of space available for driver |

These values are defined in files in the DEFS directory on the Development Package disk.

TABLES. This area contains one table for each drive that the controller handles. (The RBF manager assumes that there are as many tables as indicated by V.NDRV.) Some time after the driver Init routine is called, the RBF manager issues a request for the driver to read LSN 0 from a drive table by copying the first part of LSN 0 (up to DD.SIZ) into the table. Following is the format of each drive table:

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| DD.TOT | $00 | 3 | Number of sectors |
| DD.TKS | $03 | 1 | Track size (in sectors) |
| DD.MAP | $04 | 2 | Number of bytes in the allocation bit map |
| DD.BIT | $06 | 2 | Number of sectors per bit (cluster size) |
| DD.DIR | $08 | 3 | Address (LSN) of the root directory |
| DD.OWN | $0B | 2 | Owner’s user number |
| DD.ATT | $0D | 1 | Disk access attributes (D S PE PW PR E W R) |
| DD.DSK | $0F | 2 | Disk ID (a pseudo-random number used to detect diskette swaps) |
| DD.FMT | $10 | 1 | Media format |
| DD.SPT | $11 | 2 | Number of sectors per track. (Track 0 can use a different value specified by IT.T0S in the device descriptor.) |
| DD.RES | $13 | 2 | Reserved for future use |
| DD.SIZ | $15 | 0 | Minimum size of device descriptor |
| V.TRAK | $15 | 2 | Number of the current track (the track that the head is on, and the track updated by the driver) |
| V.BMB | $17 | 1 | Bit-map use flag: 0 = Bit map is not in use (Disk driver routines must not alter V.BMB) |
| V.FILEHD | $18 | 2 | Open file list for this drive |
| V.DISKID | $1A | 2 | Disk ID |
| V.BMAPSZ | $1C | 1 | Size of bitmap |
| V.MAPSCT | $1D | 1 | Lowest reasonable bitmap sector |
| V.RESBIT | $1E | 1 | Reserved bitmap sector |
| V.SCTKOF | $1F | 1 | Sector/track byte |
| V.SCOFST | $20 | 1 | Sector offset split from byte above |
| V.TKOFST | $22 | 4 | Reserved for future use |
| DRVMEM | $26 | . | Size of each drive table |

The format attributes (DD.FMT) are these:

Bit 0 Number of sides

0 = Single-sided

1 = Double-sided

Bit 1 Density

0 = Single-density

1 = Double-density

Bit 2 Track density

0 = Single (48 tracks per inch)

1 = Double (96 tracks per inch)

### RBF Device Driver Subroutines

Like all device driver modules, RBF device drivers use a standard executable memory module format.

The execution offset address in the module header points to a branch table that has six 3-byte entries. Each entry is typically a long branch (LBRA) to the corresponding subroutine. The branch table is defined as follows:

ENTRY	LBRA	INIT		Initialize drive

LBRA	READ	Read sector

LBRA	WRITE	Write sector

LBRA	GETSTA	Get status

LBRA	SETSTA	Set status

LBRA	TERM	Terminate device

Ensure that each subroutine exits with the C bit of the condition code register cleared if no error occurred. If an error occurs, set the C bit and return an appropriate error code in Register B.

The rest of this chapter describes the RBF device driver subroutines and their entry and exit conditions.

Init		Initializes a device and the device’s memory area.

Entry Conditions

Y	address of the device descriptor

U	address of the device memory area

Exit Conditions

CC	carry set on error

B	error code (if any)

Additional Information

  - If you want NitrOS-9 to verify disk writes, use the Request Memory system call (F$SRqMem) to allocate a 256-byte buffer area in which a sector can be read back and verified after a write.

  - You must initialize the device memory area. For floppy diskette controllers, initialization typically consists of:

- Initializing V.NDRV to the number of drives with which the controller works

- Initializing DD.TOT (in the drive table) to a non-zero value so that Sector 0 can be read or written

- Initializing V.TRAK to $FF so that the first seek finds Track 0

- Placing the IRQ service routing on the IRQ polling list, using the Set IRQ system call (F$IRQ)

- Initializing the device control registers (enabling interrupts if necessary)

  - Prior to being called, the device memory area is cleared (set to zero), except for V.PAGE and V.PORT. (These areas contain the 24-bit device address.) Ensure the driver initializes each drive table appropriately for the type of diskette that the driver expects to be used on the corresponding drive.

Read	Reads a 256-byte sector from a disk and places it in a 256-byte sector buffer.

Entry Conditions

B	MSB of the disk’s LSN

X	LSB of the disk’s LSN

Y	address of the path descriptor

U	address of the device memory area

Exit Conditions

CC	carry set on error

B	error code (if any)

Additional Information

  - The following is a typical routine for using Read:

      - Get the sector buffer address from PD.BUF in the path descriptor.

      - Get the drive number from PD.DRV in the path descriptor.

      - Compute the physical disk address from the logical sector number.

      - Initiate the Read operation

      - Copy V.BUSY to V.WAKE. The driver goes to sleep and waits for the I/O to complete. (The IRQ service routine is responsible for sending a wakeup signal.) After awakening, the driver tests V.WAKE to see if it is clear. If it is not clear, the driver goes back to sleep.

  - Whenever you read LSN 0, you must copy the first part of this sector into the proper drive table. (Get the drive number from PD.DRV in the path descriptor.) The number of bytes to copy is in DD.SIZ. Use the drive number (PD.DRV) to compute the offset for the corresponding drive table as follows:

LDA		PD.DRV,Y		Get the drive number

LDB		#DRVMEM		Get the size of a drive table

MUL

LEAX		DRVBEG,U	Get the address of the first table

LEAX		D,X			Compute the address of the table

Write	Writes a 256-byte sector buffer to a disk.

Entry Conditions

B	MSB of the disk LSN

X	LSB of the disk LSN

Y	address of the path descriptor

U	address of the device memory area

Exit Conditions

CC	carry set on error

B	error code (if any)

Additional Information

  - Following is a typical routine for using Write:

      - Get the sector buffer address from PD.BUF in the path descriptor.

      - Get the drive number from PD.DRV in the path descriptor.

      - Compute the physical disk address from the logical sector number.

      - Initiate the Write operation.

      - Copy V.BUSY to V.WAKE. The driver then goes to sleep and waits for the I/O to complete. (The IRQ service routine sends the wakeup signal.) After awakening, the driver tests V.WAKE to see if it is clear. If it is not, the driver goes back to sleep. If the disk controller cannot be interrupt-driven, it is necessary to perform a programmed I/O transfer.

      - If PF.VFY in the path descriptor is equal to zero, read the sector back in and verify that it is written correctly. Verification usually does not involve a comparison of all of the data bytes.

  - If disk writes are to be verified, the Init routine must request the buffer in which to place the sector when it is read back. Do not copy LSN 0 into the drive table when reading it back for verification.

  - Use the drive number (PD.DRV) to compute the offset to the corresponding drive table as shown for the Read routine.

GetStats and SetStats

Reads or changes device’s operating parameters.

Entry Conditions

U	address of the device memory area

Y	address of the path descriptor

A	status code

Exit Conditions

CC	carry set on error

B	error code (if any)

Additional Information

  - Get/set the device’s operating parameters (status) as specified for the Get Status and Set Status system calls. GetStat and SetStat are wild card calls.

  - It might be necessary to examine or change the register stack that contains the values of the 6809 registers at the time of the call. The address of the register stack is in PD.RGS, which is located in the path descriptor. You can use the following offsets to access any value in the register stack:

| Reg. | Relative Address | Size | 6809 Register |
| --- | --- | --- | --- |
| R$CC | $00 | 1 | Condition code register |
| R$D | $01 | 2 | Register D |
| R$A | $01 | 1 | Register A |
| R$B | $02 | 1 | Register B |
| R$DP | $03 | 1 | Register DP |
| R$X | $04 | 2 | Register X |
| R$Y | $06 | 2 | Register Y |
| R$U | $08 | 2 | Register U |
| R$PC | $0A | 2 | Program counter |

  - Register D overlays Registers A and B.

Term	Terminate a device.

Entry Conditions

U	address of the device memory area

Exit Conditions

CC	carry set on error

B	error code (if any)

Additional Information

  - This routine is called when a device is no longer in use in the system (when the link count of its device descriptor module becomes zero).

  - Following is a typical routine for using Term:

      - Wait until any pending I/O is completed.

      - Disable the device interrupts.

      - Remove the device from the IRQ polling list.

      - If the Init routine reserved a 256-byte buffer for verifying disk writes, return the memory with the Return System Memory system call (F$SRtMem).

IRQ Service Routine

Services device interrupts

Additional Information

  - The IRQ Service routine sends a wakeup signal to the process indicated by the process ID in V.WAKE when the I/O is complete. It then clears V.WAKE as a flag to indicate to the main program that the IRQ has indeed occurred.

  - When the IRQ Service routine finishes servicing an interrupt, it must clear the carry and exit with an RTS instruction.

  - Although this routine is not included in the device driver module branch table and is not called directly by the RBF manager, it is a key routine in interrupt-driven drivers. Its function is to:

      - Service the device interrupts (receive data from device or send data to it). The IRQ Service routine puts its data into and gets its data from buffers that are defined in the device memory area.

      - Wake up a process that is waiting for I/O to be completed. To do this, the routine checks to see if there is a process ID in V.WAKE (if the bit is non-zero); if so, it sends a wakeup signal to that process.

      - If the device is ready to send more data, and the out buffer is empty, disable the device’s ready to transmit interrupts.

Boot (Bootstrap Module)

Loads the boot file into RAM.

Entry Conditions

None

Exit Conditions

D	size of the boot file (in bytes)

X	address at which the boot file was loaded into memory

CC	carry set on error

B	error code (if any)

Additional Information

  - The Boot module is not part of the disk driver. It is a separate module that is stored on the boot track of the system disk with OS9P1 and REL.

  - The bootstrap module contains one subroutine that loads the bootstrap file and related information into memory. It uses the standard executable module format with a module type of $C. The execution offset in the module header contains the offset to the entry point of this subroutine.

  - The module gets the starting sector number and size of the OS9Boot file from LSN 0. NitrOS-9 allocates a memory area large enough for the Boot file. Then, it loads the Boot file into this memory area.

  - Following is a typical routine for using Boot:

      - Read LSN 0 from the disk into a buffer area. The Boot module must pick its own buffer area. LSN 0 contains the values for DD.BT (the 24-bit LSN of the bootstrap file), and DD.BSZ (the size of the bootstrap file in bytes).

      - Get the 24-bit LSN of the bootstrap file from DD.BT.

      - Get the size of the bootstrap file from DD.BSZ. The Boot module is contained in one logically contiguous block beginning at the logical sector specified in DD.BT and extending for DD.BSZ/256+1 sectors.

      - Use the NitrOS-9 Request System Memory system call (F$SRqMem) to request the memory area in which the Boot file is loaded.

      - Read the Boot file into this memory area.

      - Return the size of the Boot file and its location. Boot file is loaded.

# Sequential Character File Manager

The Sequential Character File Manager (SCFMAN) supports devices that operate on a character-by-character basis. These include terminals, printers, and modems.

SCF is a re-entrant subroutine package. The I/O manager calls the SCF manager for I/O system handling of sequential, character-oriented devices. The SCF manager includes the extensive I/O editing functions typical of line-oriented operations, such as:

- backspace

- line delete

- line repeat

- auto line feed

- screen pause

- return delay padding

The SCF-type device driver modules are CC3IO, PRINTER, and RS-232. They run the video display, printer, and serial ports respectively. See the NitrOS-9 Commands manual for additional Color Computer I/O devices.

## SCF Line Editing Functions

The SCF manager supports two sets of read and write functions. I$Read and I$Write pass data with no modification. I$ReadLn and I$WritLn provide full line editing of device functions.

### Read and Write

The Read and Write system calls to SCF-type devices correspond to the BASIC09 GET and PUT statements. While they perform little modification to the data they pass, they do filter out keyboard interrupt, keyboard terminate, and pause characters. (Editing is disabled if the corresponding character in the path descriptor contains a zero.

Carriage returns are not followed by line feeds or nulls automatically, and the high order bits are passed as sent/received.

### Read Line and Write Line

The Read Line and Write Line system calls to SCF-type devices correspond to the BASIC09 INPUT, PRINT, READ, and WRITE statements. They provide full line editing of all functions enabled for a particular device.

The system initializes I$ReadLn and I$WritLn functions when you first use a particular device. (NitrOS-9 copies the option table from the device descriptor table associated with the specific device.

Later, you can alter the calls—either from assembly-language programs (using the Get Status system call), or from the keyboard (using the TMODE command). All bytes transferred by I$ReadLn and I$WritLn have the high order bit cleared.

### SCF Definitions of the Path Descriptor

The PD.FST and PD.OPT sections of the path descriptor are reserved for an used by the SCF file manager.

The following table describes the SCF manager’s use of PD.FST and PD.OPT. For your convenience, the table also includes the other sections of the path descriptor.

The PD.OPT section contains the values that determine the line editing functions. It contains many device operating parameters that can be read or written by the Set Status or Get Status system call. Any values not set by this table default to zero.

Note: You can disable most of the editing functions by setting the corresponding control character in the path descriptor to zero. You can use the Set Status system call or the TMODE command to do this. Or, you can go a step further by setting the corresponding control character value in the device descriptor module to zero.

To determine the default settings for a specific device, you can inspect the device descriptor.

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| Universal Section (Sale for all file managers) |  |  |  |
| PD.PD | $00 | 1 | Path number |
| PD.MOD | $01 | 1 | Access mode: 1 = read 2 = write 3 = update |
| PD.CNT | $02 | 1 | Number of open images (paths using this path descriptor) |
| PD.DEV | $03 | 2 | Address of the associated device table entry |
| PD.CPR | $05 | 1 | Current process ID |
| PD.RGS | $06 | 2 | Address of the caller’s 6809 register stack |
| PD.BUF | $08 | 2 | Address of the 256-byte data buffer (if used) |

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| SCF Path Descriptor Definitions (PD.FST Section) |  |  |  |
| PD.DV2 | $0A | 2 | Device table address of the second (echo) device |
| PD.RAW | $0C | 1 | Edit flag: 0 = raw mode 1 = edit mode |
| PD.MAX | $0D | 2 | Read Line maximum character count |
| PD.MIN | $0F | 1 | Devices are mine if cleared |
| PD.STS | $10 | 2 | Status routine module address |
| PD.STM | $12 | 2 | Reserved for status routine |

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| SCF Option Section Definition (PD.OPT Section) (Copied from the device descriptor) |  |  |  |
| PD.DTP | $20 | 1 | Device class: 0 = SCF 1 = RBF 2 = PIPE 3 = SBF |
| PD.UPC | $21 | 1 | Case: 0 = uppercase and lowercase 1 = uppercase only |
| PD.BSO | $22 | 1 | Backspace: 0 = backspace 1 = backspace, space, and backspace |
| PD.DLO | $23 | 1 | Delete: 0 = backspace over line 1 = carriage return, line feed |
| PD.EKO | $24 | 1 | Echo: 0 = no echo 1 = echo |
| PD.ALF | $25 | 1 | Auto line feed: 0 = no auto line feed 1 = auto line feed |
| PD.NUL | $26 | 1 | End-of-line null count: N = number of nulls ($00) sent after each carriage return or carriage return and line feed (n = $00-$FF) |
| PD.PAU | $27 | 1 | End of page pause: 0 = no pause 1 = pause |
| PD.PAG | $28 | 1 | Number of lines per page |
| PD.BSP | $29 | 1 | Backspace character |
| PD.DEL | $2A | 1 | Delete-line character |
| PD.EOR | $2B | 1 | End-of-record character (End-of-line character) Read only. Normally set to $0D 0 = Terminate read-line only at the end of the file |
| PD.EOF | $2C | 1 | End-of-file character (read only) |
| PD.RPR | $2D | 1 | Reprint-line character |
| PD.DUP | $2E | 1 | Duplicate-last-line character |
| PD.PSC | $2F | 1 | Pause character |
| PD.INT | $30 | 1 | Keyboard-interrupt character |
| PD.QUT | $31 | 1 | Keyboard-terminate character |
| PD.BSE | $32 | 1 | Backspace-echo character |
| PD.OVF | $33 | 1 | Line-overflow character (bell CTRL-G) |
| PD.PAR | $34 | 1 | Device initialization value (parity) |
| PD.BAU | $35 | 1 | Software settable baud rate |
| PD.D2P | $36 | 2 | Offset to second device name string |
| PD.XON | $38 | 1 | ACIA XON character |
| PD.XOFF | $39 | 1 | ACIA XOFF character |
| PD.ERR | $3A | 1 | Most recent I/O error status |
| PD.TBL | $3B | 2 | Copy of device table address |
| PD.PLP | $3D | 2 | Path descriptor list pointer |
| PD.PST | $3F | 1 | Current path status |

PD.EOF specifies the end-of-file character. If this is the first and only character that is input to the SCF device, SCF returns an end-of-file error on Read or ReadLn.

PD.PSC specifies the pause character, which suspends output to the device before the next end-of-record character. The pause character also deletes any type-ahead input for ReadLn.

PD.INT specifies the keyboard-interrupt character. When the character is received, the system sends a keyboard-terminate signal to the last user of a path. The character also terminates the current I/O request (if any) with an error identical to the keyboard interrupt signal code.

PD.QUT specifies the keyboard-terminate character. When this character is received, the system sends a keyboard-terminate signal to the last user of a path. The system also cancels the current I/O request (if any) by sending an error code identical to the keyboard interrupt signal code.

PD.PAR specifies the parity information for external serial devices.

PD.BAU specifies baud rate, word length, and stop bit information for serial devices.

PD.XON contains either the character used to enable transmission of characters or a null character that disables the use of XON.

PD.XOFF contains either the character used to disable transmission of characters or a null character that disables the use of XOFF.

## SCF-Type Device Descriptor Modules

The following chart shows how the initialization table in the device descriptors is used for SCF-type devices. The values are those the I/O manager copies from the device descriptor to the path descriptor.

An SCF editing function is turned off if its corresponding value is set to zero. For example, if IT.EOF is set to zero, there is no end-of-file character.

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| (header) | $00-$11 |  | Standard device descriptor module header |
| IT.DVC | $12 | 1 | Device class: 0 = SCF 1 = RBF 2 = PIPE 3 = SBF |
| IT.UPC | $13 | 1 | Case: 0 = upper- and lowercase 1 = uppercase only |
| IT.BSO | $14 | 1 | Backspace: 0 = backspace 1 = backspace, space, and backspace |
| IT.DLO | $15 | 1 | Delete: 0 = backspace over line 1 = carriage return |
| IT.EKO | $16 | 1 | Echo: 0 = echo off 1 = echo on |
| IT.ALF | $17 | 1 | Auto line feed: 0 = auto line feed disabled 1 = auto line feed enabled |
| IT.NUL | $18 | 1 | End-of-line null count |
| IT.PAU | $19 | 1 | Pause: 0 = end-of-page pause disabled 1 = end-of-page pause enabled |
| IT.PAG | $1A | 1 | Number of lines per page |
| IT.BSP | $1B | 1 | Backspace character |
| IT.DEL | $1C | 1 | Delete-line character |
| IT.EOR | $1D | 1 | End-of-record character |
| IT.EOF | $1E | 1 | End-of-file character |
| IT.RPR | $1F | 1 | Reprint-line character |
| IT.DUP | $20 | 1 | Duplicate-last-line character |
| IT.PSC | $21 | 1 | Pause character |
| IT.INT | $22 | 1 | Interrupt character |
| IT.QUT | $23 | 1 | Quit character |
| IT.BSE | $24 | 1 | Backspace echo character |
| IT.OVF | $25 | 1 | Line-overflow character (bell) |
| IT.PAR | $26 | 1 | Initialization value—used to initialize a device control register when a path is opened to it (parity) |
| IT.BAU | $27 | 1 | Baud rate |
| IT.D2P | $28 | 2 | Attached device name string offset |
| IT.XON | $2A | 1 | X-ON character |
| IT.XOFF | $2B | 1 | X-OFF character |
| IT.COL | $2C | 1 | Number of columns for display |
| IT.ROW | $2D | 1 | Number of rows for display |
| IT.WND | $2E | 1 | Window number |
| IT.VAL | $2F | 1 | Data in rest of descriptor is valid |
| IT.STY | $30 | 1 | Window type |
| IT.CPX | $31 | 1 | X cursor position |
| IT.CPY | $32 | 1 | Y cursor position |
| IT.FGC | $33 | 1 | Foreground color |
| IT.BGC | $34 | 1 | Background color |
| IT.BDC | $35 | 1 | Border color |

## SCF-Type Device Driver Modules

An SCF-type device driver module contains a package of subroutines that perform raw (unformatted) data I/O transfers to or from a specific hardware controller. Such a module is usually re-entrant so that one copy of the module can simultaneously run several devices that use identical I/O controllers. The I/O manager allocates a permanent memory area for each controller sharing the driver.

The size of the memory area is defined in the device driver module header. The I/O manager and SCF use some of the memory area. The device driver can use the rest in any way (typically as variables and buffers). Typically, the driver uses the area as follows:

| Name | Relative Address | Size (Bytes) | Use |
| --- | --- | --- | --- |
| V.PAGE | $00 | 1 | Port extended 24-bit address |
| V.PORT | $01 | 2 | Device base address (defined by the I/O manager) |
| V.LPRC | $03 | 1 | ID of the last active process |
| V.BUSY | $04 | 1 | ID of the active process (defined by RBF): 0 = no active process |
| V.WAKE | $05 | 1 | ID of the process to reawaken after the device completes I/O (defined by the device driver): 0 = no waiting process |
| V.USER | $06 | 0 | Beginning of file manager specific storage |
| V.TYPE | $06 | 1 | Device type or parity |
| V.LINE | $07 | 1 | Lines left until the end of the page |
| V.PAUS | $08 | 1 | Pause request: 0 = no pause requested |
| V.DEV2 | $09 | 2 | Attached device memory area |
| V.INTR | $0B | 1 | Interrupt character |
| V.QUIT | $0C | 1 | Quit character |
| V.PCHR | $0D | 1 | Pause character |
| V.ERR | $0E | 1 | Error accumulator |
| V.XON | $0F | 1 | XON character |
| V.XOFF | $10 | 1 | XOFF character |
| V.KANJI | $11 | 1 | Reserved |
| V.KBUF | $12 | 2 | Reserved |
| V.MODADR | $14 | 2 | Reserved |
| V.PDLHD | $16 | 2 | Path descriptor list header |
| V.RSV | $18 | 5 | Reserved |
| V.SCF | $1D | 0 | End of SCF memory requirements |
| FREE | $1D | 0 | Free for the device driver to use |

V.LPRC contains the process ID of the last process to use the device. The IRQ service routing sends this process the proper signal if it receives a quit character or an interrupt character. V.LPRC is defined by SCF.

V.BUSY contains the process ID of the process that is using the device. (If the device is not being used, V.BUSY contains a zero.) The process ID is used by SCF to prevent more than one process from using the device at the same time. V.BUSY is defined by SCF.

### SCF Device Driver Subroutines

Like all device drivers, SCF device drivers use a standard executable memory module format.

The execution offset address in the module header points to a branch table that has six 3-byte entries. Each entry is typically an LBRA to the corresponding subroutine. The branch table is defined as follows:

ENTRY	LBRA	INIT		Initialize driver

LBRA	READ	Read character

LBRA	WRITE	Write character

LBRA	GETSTA	Get status

LBRA	SETSTA	Set status

LBRA	TERM	Terminate device

If no error occurs, each subroutine exits with the C bit in the Condition Code register cleared. If an error occurs, each subroutine sets the C bit and returns an appropriate error code in Register B.

The rest of this chapter describes these subroutines and their entry and exit conditions.

Init	Initializes device control registers and enables interrupts if necessary.

Entry Conditions

Y	address of the device descriptor

U	address of the device memory area

Exit Conditions

CC	carry set on error

B	error code (if any)

Additional Information

  - Prior to being called, the device memory area is cleared (set to zero), except for V.PAGE and V.PORT. (V.PAGE and V.PORT contain the device address.) There is no need to initialize the part of the memory area used by the I/O manager and SCF.

  - Follow these steps to use Init:

      - Initialize the device memory area.

      - Place the IRQ service routine on the IRQ polling list, use the Set IRQ system call (F$IRQ).

      - Initialize the device control registers.

Read	Reads the next character from the input buffer.

Entry Conditions

Y	address of the path descriptor

U	address of the device memory area

Exit Conditions

A	character read

CC	carry set on error

B	error code (if any)

Additional Information

  - This is a step by step description of a Read operation:

      - Read gets the next character from the input buffer.

      - If no data is ready, Read copies its process ID from V.BUSY into V.WAKE. It then uses the Sleep system call to put itself to sleep.

      - Later, when Read receives data, the IRQ service routine leaves the data in a buffer. Then, the routine checks V.WAKE to see if any process is waiting for the device to complete I/O. If so, the IRQ service routine sends a wakeup signal to the waiting process.

  - Data buffers are not automatically allocated. If a buffer is used, it defines it in the device memory area.

Write	Sends a character (places a data byte in an output buffer) and enables the device output interrupts.

Entry Conditions

A	character to write

Y	address of the path descriptor

U	address of the device memory area

Exit Conditions

CC	carry set on error

B	error code (if any)

Additional Information

  - If the data buffer is full, Write copies its process ID from V.BUSY into V.WAKE. Write then puts itself to sleep.

  - Later, when the IRQ service routine transmits a character and makes room for more data, it checks V.WAKE to see if there is a process waiting for the device to complete I/O. If there is, the routine sends a wakeup signal to that process.

  - Write must ensure that the IRQ service routine that starts it begins to place data in the buffer. After an interrupt is generated, the IRQ service routine continues to transmit data until the data buffer is empty. Then, it disables the device’s ready-to-transmit interrupts.

  - Data buffers are not allocated automatically. If a buffer is used, define it in the device memory area.

GetSta and SetSta

Gets/sets device operating parameters (status) as specified for the Get Status and Set Status system calls. GetSta and SetSta are wildcard calls.

Entry Conditions

A	depends on the function code

Y	address of the path descriptor

U	address of the device memory area

Other registers depend on the function code.

Exit Conditions

CC	carry set on error

B	error code (if any)

Other registers depend on the function code

Additional Information

  - Any codes not defined by the I/O manager or SCF are passed to the device driver.

  - You might need to examine or change the register stack that contains the values of the 6809 registers at the time of the call. The address of the register stack can be found in PD.RGS, which is located in the path descriptor.

  - You can use the following offsets to access any value in the register packet:

| Reg. | Relative Address | Size | 6809 Register |
| --- | --- | --- | --- |
| R$CC | $00 | 1 | Condition code register |
| R$D | $01 | 2 | Register D |
| R$A | $01 | 1 | Register A |
| R$B | $02 | 1 | Register B |
| R$DP | $03 | 1 | Register DP |
| R$X | $04 | 2 | Register X |
| R$Y | $06 | 2 | Register Y |
| R$U | $08 | 2 | Register U |
| R$PC | $0A | 2 | Program counter |
| The function code is retrieved from R$B on the user stack. |  |  |  |

Term	Terminates a device. Term is called when a device is no longer in use (when the link count of the device descriptor module becomes zero).

Entry Conditions

U	pointer to the device memory area

Exit Conditions

CC	carry set on error

B	error code (if any)

Additional Information

  - To use Term:

      - Wait until the IRQ service routine empties the output buffer.

      - Disable the device interrupts.

      - Remove the device from the IRQ polling list.

  - When Term closes the last path to a device, NitrOS-9 returns to the memory pool the memory that the device used. If the device has been attached to the system using the I$Attach system call, NitrOS-9 does not return the static storage for the driver until an I$Detach call is made to the device. Modules contained in the Boot file are never terminated, even if their link counts reach zero.

IRQ Service Routine

Receives device interrupts. When I/O is complete, the routine sends a wakeup signal to the process identified by the process ID in V.WAKE. The routine also clears V.WAKE as a flag to indicate to the main program that the IRQ has occurred.

Additional Information

  - The IRQ Service Routine is not included in the device driver branch tables, and is not called directly by SCF. However, it is a key routine in device drivers.

  - When the IRQ Service routine finishes servicing an interrupt, the routine must clear the carry and exit with an RTS instructions.

  - Here is a typical sequence of events that the IRQ Service Routing performs:

      - Service the device interrupts (receive data from the device or send data to it). Ensure this routine puts its data into and gets its data from buffers that are defined in the device memory area.

      - Wake up any process that is waiting for I/O to complete. To do this, the routine checks to see if there is a process ID in V.WAKE (a value other than zero); if so, it sends a wakeup signal to that process.

      - If the device is ready to send more data, and the output buffer is empty, disable the device’s ready-to-transmit interrupts.

      - If a pause character is received, set V.PAUS in the attached device storage area to a value other than zero. The address of the attached device memory area is in V.DEV2.

      - If a keyboard terminate or interrupt character is received, signal the process in V.LPRC (last known process) if any.

# The Pipe File Manager (PIPEMAN)

The Pipe file manager handles control or processes that use paths to pipes. Pipes allow concurrently executing processes to send each other data by using the output of one process (the writer) as input to a second process (the reader). The reader gets input from the standard input. The exclamation point (!) operator specifies that the input or output is from or to a pipe. The Pipe file manager allocates a 256-byte block and a path descriptor for data transfer. The Pipe file manager also determines which process has control of the pipe. The Pipe file manager has the standard file manager branch table at its entry point:

ENTRY	LBRA	Create

LBRA	Open

LBRA	MakDir

LBRA	ChgDir

LBRA	Delete

LBRA	Seek

LBRA	PRead

LBRA	PWrite

LBRA	PRdLn

LBRA	PWrLn

LBRA	GetStat

LBRA	SetStat

LBRA	Close

You cannot use MakDir, ChgDir, Delete, and Seek with pipes. If you try to do so, the system returns E$UNKSVC (unknown service request). GetStat and SetStat are also no-action service routines. They return without error.

Create and Open perform the same functions. They set up the 256-byte data exchange buffer and save several addresses in the path descriptor.

The Close request checks to see if any process is reading or writing through the pipe. If not, NitrOS-9 returns the buffer.

PRead, PWrite, PRdLn, and PWrLn read data from the buffer and write data to it.

The ! operator tells the Shell that processes wish to communicate through a pipe. For example:

poc1 ! proc2

In this example, shell forks Proc1 with the standard output path to a pipe and forks Proc2 with the standard input path from a pipe.

Shell can also handle a series of processes using pipe. For example:

proc1 ! proc2 ! proc3 ! proc4

The following outline shows how to set up pipes between processes:

Open /pipe		save path in variable x

Dup path #1	save stdout in variable y

Close #1		make path available

Dup x		put pipe in stdout

(Dup uses lowest available)

Fork proc1		fork process 1

Close #1		make path available

Dup y		restore stdout

Close y		make path available

Dup path #0	save stdin in Y

Close #0		make path available

Dup x		put pipe in stdin

Fork proc2		fork process 2

Close #0		make path available

Dup y		restore stdin

Close x		no longer needed

Close y		no longer needed

Example: The following example shows how an application can initiate another process with the stdin and stdout routed through a pipe:

Open /pipe1	save path in variable a

Open /pipe2	save path in variable b

Dup 0		save stdin in variable x

Dup 1		save stdout in variable y

Close #0		make stdin path available

Close #1		make stdout path available

Dup a		make pipe1 stdin

Dup b		make pipe2 stdout

Fork new process

Close #0		make stdin path available

Close #1		make stdout path available

Dup x		restore stdin

Dup y		restore stdout

Return a&b		return pipe path numbers to caller

# System Calls

System calls are used to communicate between the NitrOS-9 operating system and assembly-language programs. There are two major types of calls—I/O calls and function calls.

Function calls include user mode calls and system mode calls.

Each system call has a mnemonic name. Names of I/O calls start with I$. For example, the Change Directory call is I$ChgDir. Names of function calls start with F$. For example, the Allocate Bits call is F$AllBit. The names are defined in the assembler-input conditions equate file called OS9Defs.

System mode calls are privileged. You can execute them only while NitrOS-9 is in the system state (when it is processing another system call, executing a file manager or device driver, and so on).

System mode calls are included in this manual primarily for programmers writing device drivers and other system-level applications.

## Calling Procedure

To execute any system calls, you need to use an SWI2 instruction:

- Load the 6809 registers with any appropriate parameters.

- Execute an SWI instruction, followed immediately by a constant byte, which is the request code. In the references in this chapter, the first line is the system call name (for example Close Path) and the second line contains the call’s mnemonic name (for example I$Close), the software interrupt Code 2 (103F), and the call’s request code (for example, 8F) in hexadecimal.

- After NitrOS-9 processes the call, it returns any parameters in the 6809 registers. If an error occurs, the C bit of the condition code register is set and Register B contains the appropriate error code. This feature permits a BCS or BCC instruction immediately following the system call to branch either if there is an error or if no error occurs.

As an example, here is the Close system call:

LDA		PATHNUM

SWI2

FCB		$8F

BCS		ERROR

You can use the assembler’s OS9 directive to simplify the call, as follows:

LDA		PATHNUM

OS9		I$Close

BCS		ERROR

The ASM assembler allows any combination of upper- or lowercase letters. The RMA assembler, included in the OS-9 Level Two Development Pak, is case sensitive. The names in this manual have been spelled with upper and lower case letters, matching the defs for RMA.

## I/O System Calls

NitrOS-9’s I/O calls are easier to use than many other systems’ I/O calls. This is because the calling program does not have to allocate and set up file control blocks, sector buffers, and so on.

Instead, NitrOS-9 returns a 1-byte path number whenever a process opens a path to a file or device. Until the path is closed, you can use this path number in later I/O requests to identify the file or device.

In addition, NitrOS-9 allocates and maintains its own data structures; so, you need not deal with them.

## System Call Descriptions

The rest of this chapter consists of the system call descriptions. At the top of each description is the system call name, followed by its mnemonic name, the SWI2 code, and the request code. Next are the call’s entry and exit conditions, followed by additional information about the code where appropriate.

In the system call descriptions, registers not specified as entry or exit conditions are not altered. Strings passed as parameters are normally terminated with a space character and end-of-line character, or with Bit 7 of the last character set.

If an error occurs on a system call, the C bit of Register CC is set and Register B contains the error code. If no error occurs, the C bit is clear and Register B contains a value of zero.

## User Mode System Calls Quick Reference

Following is a summary of the User Mode System Calls referenced in this chapter:

F$AllBit	Sets bits in an allocation bit map

F$Chain	Chains a process to a new module

F$CmpNam	Compares two names

F$CpyMem	Copies external memory

F$CRC	Generates a cyclic redundancy check

F$DelBit	Deallocates bits in an allocation bit map

F$Exit	Terminates a process

F$Fork	Starts a new process

F$GBlkMp	Gets a copy of a system block map

F$GPrDsc	Gets a copy of a process descriptor

F$Icpt	Set a signal intercept trap

F$ID	Returns a process ID

F$Link	Links to a memory module

F$Load	Loads a module from mass storage

F$Mem	Changes a process’s data area size

F$NMLink	Links to a module; does not map the module into the user’s address space

F$NMLoad	Loads a module but does not map it into the user’s address space

F$PErr	Prints an error message

F$PrsNam	Parses a pathlist name

F$SchBit	Searches a bit map

F$Send	Sends a signal to a process

F$Sleep	Suspends a process

F$SPrior	Sets a process’s priority

F$SSWI	Sets a software interrupt vector

F$STime	Sets a system time

F$SUser	Sets a user ID number

F$Time	Returns the current time

F$Unlink	Unlinks a module

F$Unload	Unlinks a module by name

F$Wait	Waits for a signal

I$Attach	Attaches to an I/O device

I$ChgDir	Changes a working directory

I$Close	Closes a path

I$Create	Creates a new file

I$Delete	Deletes a file

I$DeletX	Deletes a file from the execution directory

I$Detach	Detaches an I/O device

I$Dup	Duplicates a path

I$GetStt	Gets a device’s status

I$MakDir	Creates a directory file

I$Open	Opens a path to an existing file

I$Read	Reads data from a device

I$ReadLn	Reads a line of data from a device

I$Seek	Positions a file pointer

I$SetStt	Sets a device’s status

I$Write	Writes data to a device

I$WritLn	Writes a data line to a device

## System Mode Calls Quick Reference

Following is a summary of the System Mode Calls referenced in this chapter:

F$Alarm	Sets up an alarm

F$All64	Allocates a 64-byte memory block

F$AllHRAM	Allocates high RAM

F$AllImg	Allocates image RAM blocks

F$AllPrc	Allocates a process descriptor

F$AllRAM	Allocates RAM blocks

F$AllTsk	Allocates a process task number

F$AProc	Enters active process queue

F$Boot	Performs a system bootstrap

F$BtMem	Performs a memory request bootstrap

F$ClrBlk	Clears the specified block of memory

F$DATLog	Converts a DAT block offset to a logical address

F$DelImg	Deallocates image RAM blocks

F$DelPrc	Deallocates a process descriptor

F$DelRAM	Deallocates RAM blocks

F$DelTsk	Deallocates a process task number

F$ELink	Links modules using a module directory entry

F$FModul	Finds a module directory entry

F$Find64	Finds a 64-byte memory block

F$FreeHB	Gets a free high block

F$FreeLB	Gets a free low block

F$GCMDir	Compacts module directory entries

F$GProcP	Gets a process’s pointer

F$IODel	Deletes an I/O module

F$IOQu	Puts an entry into an I/O queue

F$IRQ	Makes an entry into IRQ polling table

F$LDABX	Loads Register A from 0,X in Task B

F$LDAXY	Loads A[X,[Y]]

F$LDDDXY	Loads D[D+X,[Y]]

F$MapBlk	Maps the specified blocks

F$Move	Moves data to a different address space

F$NProc	Starts the next process

F$RelTsk	Releases a task number

F$ResTsk	Reserves a task number

F$Ret64	Returns a 64-byte memory block

F$SetImg	Sets a process DAT image

F$SetTsk	Sets a process’s task DAT registers

F$Slink	Performs a system link

F$SRqMem	Performs a system memory request

F$SRtMem	Performs a system memory return

F$SSvc	Installs a function request

F$STABX	Stores Register A at 0,X in Task B

F$VIRQ	Makes an entry in a virtual IRQ polling table

F$VModul	Validates a module

## User System Calls

Allocate Bits	Sets bits in an allocation bit map

OS9 F$AllBit 103F 13

Entry Conditions

D	number of the first bit to set

X	starting address of the allocation bit map

Y	number of bits to set

Error Output

CC	carry set on error

B	error code (if any)

Additional Information

- Bit numbers range from 0 to n-1, where n is the number of bits in the allocation bit map.

- Warning: Do not issue the Allocate Bits call with Register Y ste to 0 (a bit count of 0).

## I/O User System Calls

## Privileged System Mode Calls

## Get Status System Calls

## Set Status System Calls

# Appendices

## A. System Module Diagrams

## B. Standard Floppy Disk Format

## C. System Error Codes

Index

device descriptor, 7

device driver, 7

equate file, 10

file manager, 7

kernel, 6

module

directory, 11

link count, 11

memory, 11

primary, 17

process, 17

active, 17

active queue, 19

descriptor, 17, 18

ID, 18

state, 18

system call, 10

system calls

function, 11

I/O, 11

privileged, 11

tick, 6

time slice, 17

user ID, 18