DriveWire 3 Protocol

Document Revision B

Boisy G. Pitre

November 29, 2009

Introduction

The DriveWire 3 Protocol Specification defines a set of communication messages that allow the Tandy/Radio Shack Color Computer (CoCo) to utilize the storage capacity of a modern personal computer (Server).  This storage model gives the CoCo the appearance that it is directly connected to a large storage device, when in fact, connectivity is achieved using a serial cable connected from the Server to the CoCo’s built-in “Serial I/O” port.

There are distinct advantages in utilizing DriveWire 3 as a storage medium:

- Cost and space savings: no additional hardware is needed for a CoCo to use mass storage; no floppy controller, hard drive controller or hard drives are required.

- Remote operation: the serial cable that tethers the CoCo to the Server can extend for some length, allowing the CoCo to be positioned a considerable distance from the Server.

- Easy data management: virtual disks that reside on the Server can be easily copied, emailed or archived for backup purposes.

The essence of communication between the CoCo and Server is a documented set of uni- and bi-directional messages called transactions.  Each transaction is composed of one or more packets, which are passed between CoCo and Server through a serial line connection.  Using clever timing techniques on the CoCo, baud rates of up to 115,200 are reliably achievable.

System Requirements

The Server shall be defined as a modern personal computer or hand-held device with sufficient mass storage and an RS-232 serial port that can be reliably driven at rates of up to 115,200 bits per second.

The CoCo shall be defined as any model of Color Computer produced by Tandy/Radio Shack with at least 16K of RAM, as well as an appropriate operating environment that provides mass storage services (such as Disk BASIC or OS-9).

Physical Interface Requirements

The physical cable connecting the CoCo to the Server is recommended to be no longer than 10 feet in length.  At a minimum, it shall carry the following four wires:

- Ground

- Vcc

- RD (Read Data)

- WD (Write Data)

A 4 pin DIN connector shall be on one end of the cable which will mate to the jack on the back of the CoCo marked “Serial I/O”.  The other end of the cable shall be either a DB-9 or DB-25 female connector which will mate to an appropriate serial port on the Server.

The following table shows the connections between the CoCo 4 pin DIN connector to either a 9-pin DB-9 serial connector or a 25-pin DB-25 serial connector.

| CoCo 4 pin DIN | DB-9 OR | DB-25 |
| --- | --- | --- |
| Pin 2 (RD) | Pin 3 (TD) | Pin 2 (TD) |
| Pin 3 (GND) | Pin 5 (GND) | Pin 7 (GND) |
| Pin 4 (TD) | Pin 2 (RD) | Pin 3 (RD) |

Table 1 - CoCo to Server Cable Pinout

Communication Protocol

The communication protocol shall follow the RS-232 standard of 8-N-1 (1 start bit, 8 data bits, and 1 stop bit).  For Color Computer models 1 and 2, the supported clock speed shall be .89MHz and the bit rate shall be 57,600 bits per second.  For the Tandy Color Computer 3, the supported clock speed shall be 1.78MHz and the bit rate shall be 115,200 bits per second.

Transactions

The transaction is the basis for exchanging information between CoCo and Server.  All transactions consist of one or more packets of information, and are generally initiated from the CoCo.

Some transactions are uni-directional: from the CoCo to the Server.  Other transactions are bi-directional.  In a bi-directional transaction, both the CoCo and the Server shall send their respective responses within 250 milliseconds (1/4 of a second) of receiving the last packet from its peer, or else each shall presume a timeout condition and abort the transaction.

With few exceptions, all CoCo initiated packets have the following common header:

| Byte | Value |
| --- | --- |
| 0 | Command Code |

Table 2 – Common Transaction Header

The command code is a byte value that communicates to the Server the desired command to be executed.

The Initialization Transaction

This is a uni-directional transaction, and is performed by the CoCo as a notification to the Server when its driver software is initialized. The following Initialization packet is sent to the Server:

| Byte | Value |
| --- | --- |
| 0 | OP_INIT ($49) |

Table 3 - Initialization Packet

Upon receipt of the packet, the Server shall perform the following steps:

- If not already done, open the disk images for all supported drives.

The Read Extended Transaction

This bi-directional transaction is initiated by the CoCo to request one 256 byte sector of data from the Server.  When the CoCo desires to request a sector, it will send the following Read Extended Request packet:

| Byte | Value |
| --- | --- |
| 0 | OP_READEX ($D2) |
| 1 | Drive number |
| 2 | Bits 23-16 of LSN |
| 3 | Bits 15-8 of LSN |
| 4 | Bits 7-0 of LSN |

Table 4 - Read Request Packet

The drive number is a value between 0-255, and represents the disk image where the desired sector is located.  The LSN is the Logical Sector Number, and is a 24 bit value.

Read Extended Packet

Upon receipt of the packet, the Server shall consult the appropriate disk image for the provided LSN and read that 256 byte sector.  If an error occurs and the Server cannot read the specified sector, it shall return the Read Failure packet:

| Byte | Value |
| --- | --- |
| 0-255 | 256 bytes of zeroed sector data |

Table 5 – Read Failure Packet

If no error occurred on reading the sector from the Server file system, the Server shall return the following packet:

| Byte | Value |
| --- | --- |
| 0-255 | 256 bytes of valid sector data |

Table 6 – Read Success Packet

Upon receipt of either the Read Success or the Read Failure packet, the CoCo shall:

- Compute the 16 bit checksum on the 256 bytes of sector data received from the Server.

- Send the computed checksum to the Server as follows:

| Byte | Value |
| --- | --- |
| 0 | Bits 15-8 of the checksum (computed by CoCo) |
| 1 | Bits 7-0 of the checksum (computed by CoCo) |

Table 7 – CoCo Checksum Packet

Upon receipt of the checksum packet, the Server shall take one of two actions:

Action 1 – No Error Condition

If the server acquired the requested sector without error, then it shall compute the 16 bit checksum on the 256 bytes of sector data sent to the CoCo and compare its calculated checksum against that of the CoCo.

If the compare yields a match occurs, then Server shall send the following packet to the CoCo:

| Byte | Value |
| --- | --- |
| 0 | No error code (0) |

Table 8 – Checksum Match Packet

If the compare does not yield a match, then Server shall send the following packet to the CoCo:

| Byte | Value |
| --- | --- |
| 0 | Checksum mismatch (243) |

Table 9 – Checksum Mismatch Packet

Once either of these packets is received by the CoCo, the Read transaction is considered terminated.

Action 2 – Error Condition

If the Server was unable to acquire the requested sector due to an error, then the Server shall return an appropriate read error code:

| Byte | Value |
| --- | --- |
| 0 | Read error code (1-242 or 244-255) |

Table 10 – Read Error Packet

Once the Read Error packet is received by the CoCo, the Read transaction is considered terminated.

See Appendix A for a list of supported error codes, and Appendix B for the checksum algorithm used by the CoCo to generate the checksums.

ReRead Extended Packet

In the event that the CoCo receives a Checksum Mismatch packet during the Read Transaction, the CoCo MAY send the following packet to the Server:

| Byte | Value |
| --- | --- |
| 0 | OP_REREADEX ($F2) |
| 1 | Drive number |
| 2 | Bits 23-16 of LSN |
| 3 | Bits 15-8 of LSN |
| 4 | Bits 7-0 of LSN |

Table 11 – ReRead Extended Request Packet

Upon receiving this packet, the Server will attempt a retransmission of the requested sector.

At any time, the CoCo may suspend the re-reading of a sector if errors continue to accumulate, and the server will return to monitoring for additional commands.

The Write Transaction

This bi-directional transaction allows the CoCo to send one 256 byte sector of data to the Server in order for it to be written to a specific virtual drive.

| Byte | Value |
| --- | --- |
| 0 | OP_WRITE ($57) |
| 1 | Drive number |
| 2 | Bits 23-16 of LSN |
| 3 | Bits 15-8 of LSN |
| 4 | Bits 7-0 of LSN |
| 5-260 | 256 bytes of sector data to write |
| 261 | Bits 15-8 of checksum (computed by CoCo) |
| 262 | Bits 7-0 of checksum (computed by CoCo) |

Table 12 - Write Request Packet

The Server shall implement a timeout of 250 milliseconds (1/4 second), in the event that the CoCo stops sending data. This allows the Server to recover from the Write Transaction state and return to processing other packets.

Once the Server receives this packet, it shall:

- Compute the 16 bit checksum over the 256 byte sector it received

- Compare the checksum that it computed against the checksum provided by the CoCo

Once the checksums have been compared, the Server shall acknowledge the transmission as follows:

| Byte | Value |
| --- | --- |
| 0 | 0 (checksum OK) |

Table 13 – Write Success Packet

The CoCo, upon receipt of the packet, will do one the following:

- If the error code received is 0 ($00), the CoCo will assume the transfer of the sector was a success and terminate the transmission.

- If the error code received is 243 (E$CRC), the CoCo MAY send the following packet:

| Byte | Value |
| --- | --- |
| 0 | OP_REWRITE ($77) |
| 1 | Drive number |
| 2 | Bits 23-16 of LSN |
| 3 | Bits 15-8 of LSN |
| 4 | Bits 7-0 of LSN |
| 5-260 | 256 bytes of sector data to write |
| 261 | Bits 15-8 of checksum (computed by CoCo) |
| 262 | Bits 7-0 of checksum (computed by CoCo) |

Table 14 - ReWrite Request Packet

And the Write Transaction will commence again.

GetStat/SetStat Transaction

This uni-directional transaction is for informational purposes only and may not be supported in all client environments.  Its inclusion into the protocol specification is mainly for the benefit of DriveWire-enabled OS-9 drivers that execute on the CoCo.

In such a driver, all GetStats/SetStats are passed to the Server via the OP_GETSTAT or OP_SETSTAT code, followed by a byte representing the GetStat/SetStat code. The Server may wish to log this information, but does not act upon the packet itself.

| Byte | Value |
| --- | --- |
| 0 | OP_GETSTAT ($47) or OP_SETSTAT ($53) |
| 1 | Drive number |
| 2 | GetStat or SetStat code |

Table 15 – GetStat/SetStat Packet

Termination Transaction

Upon termination of the DriveWire client on the CoCo, this uni-directional transaction shall be sent to the Server:

| Byte | Value |
| --- | --- |
| 0 | OP_TERM ($54) |

Table 16 – Termination Packet

Upon receipt of the packet, the Server should:

- Close the disk file associated for all supported drives.

The Reset Transaction

When the CoCo’s reset button is pressed, the state of the serial port changes such that byte $FE (OP_RESET1) or $FF (OP_RESET2) will be present on the serial line. This condition is therefore considered a uni-directional transaction to the Server to indicate a global reset:

| Byte | Value |
| --- | --- |
| 0 | OP_RESET1 ($FE) or OP_RESET2 ($FF) |

Table 17 - Reset Packet

Upon receipt of the packet, the Server shall:

- Reset any statistical values related to read/write and sector transfer throughput for ALL drives.

- Flush all caches connected to the virtual drives.

No-Op Transaction

This is a uni-directional transaction that can be ignored by the Server.

| Byte | Value |
| --- | --- |
| 0 | OP_NOP ($00) |

Table 18 – No Operation Packet

Time Transaction

This bi-directional transaction allows the CoCo to request time and date information from the Server:

| Byte | Value |
| --- | --- |
| 0 | OP_TIME ($23) |

Table 19 – Time Request Packet

Upon receipt of the packet, the Server shall return the following packet to the

CoCo:

| Byte | Value |
| --- | --- |
| 0 | Current Year minus1900 (0-255) |
| 1 | Month (1-12) |
| 2 | Day (0-31) |
| 3 | Hour (0-23) |
| 4 | Minute (0-59) |
| 5 | Second (0-59) |
| 6 | Day of Week (0-6; 0 = Sunday) |

Table 20 – Time Delivery Packet

WIREBUG

DriveWire 3 introduces a special set of transactions to facilitate remote debugging between a CoCo and the Server that it is connected to.  This feature, known as WireBug, allows the transfer of register information and memory from the CoCo to the Server.  Once the CoCo has entered WireBug mode, it can receive messages initiated by the server.

Because of the nature of the protocol and to increase accuracy and efficiency, WireBug uses a fixed packet size of 24 bytes and reserves the last byte as an 8-bit checksum byte.

Enter WireBug Mode Transaction

Using this uni-directional transaction, the CoCo notifies the Server that it has entered WireBug remote debugging mode.

| Byte | Value |
| --- | --- |
| 0 | OP_WIREBUG_MODE ($42) |
| 1 | CoCo Type ($02 = CoCo 1/2, $03 = CoCo 3) |
| 2 | CPU Type ($08 = 6809, $03 = 6309) |
| 3-23 | Reserved for future definition |

Table 21 – Enter WireBug Mode Packet

Once the CoCo has sent this packet, it shall go into WireBug Mode where it will wait for commands from the Server.

Along with notifying the Server that the CoCo is in WireBug mode, the packet also communicates the CoCo and processor type.  The Server should use this information as a cue to what type of processor is in the CoCo, and adjust accordingly.

Read Registers Transaction (Server Initiated)

Using this bi-directional transaction, the Server requests the contents of the registers from the CoCo.

| Byte | Value |
| --- | --- |
| 0 | OP_WIREBUG_READREGS ($52) |
| 1-22 | 0 |
| 23 | Checksum |

Table 22 – WireBug Read Registers Packet

Upon receipt of the packet, the CoCo shall compute the checksum of bytes 0-22 and compare its computed value against byte 23. If a checksum match does not occur, then the CoCo shall disregard the packet and send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | Checksum mismatch (243) |

Table 23 – Checksum Mismatch Packet

Upon receipt of this packet, the Server may elect to restart the transaction.

If a match occurs, the CoCo shall respond with the following packet:

| Byte | Value |
| --- | --- |
| 0 | OP_WIREBUG_READREGS ($52) |
| 1 | Value of DP register |
| 2 | Value of CC register |
| 3 | Value of A register |
| 4 | Value of B register |
| 5 | Value of E register |
| 6 | Value of F register |
| 7 | Hi-byte of X register |
| 8 | Lo-byte of X register |
| 9 | Hi-byte of Y register |
| 10 | Lo-byte of Y register |
| 11 | Hi-byte of U register |
| 12 | Lo-byte of U register |
| 13 | Value of MD register |
| 14 | Hi-byte of V register |
| 15 | Lo-byte of V register |
| 16 | Hi-byte of SP register |
| 17 | Lo-byte of SP register |
| 18 | Hi-byte of PC register |
| 19 | Lo-byte of PC register |
| 20-22 | Don’t Care |
| 23 | Checksum |

Table 24 – WireBug Read Registers Response Packet

Upon receipt of the packet, the Server shall compute the checksum of bytes 0-22 and compare its computed value against byte 23.  If a match occurs, the Server shall accept the packet’s payload.  If a match does not occur, then the Server may elect to restart the transaction.

Write Registers Transaction (Server Initiated)

Using this bi-directional transaction, the Server requests that the contents of the packet passed to the CoCo be written to its registers.

The following packet shall be sent to the CoCo:

| Byte | Value |
| --- | --- |
| 0 | OP_WIREBUG_WRITEREGS ($72) |
| 1 | Value of DP register |
| 2 | Value of CC register |
| 3 | Value of A register |
| 4 | Value of B register |
| 5 | Value of E register |
| 6 | Value of F register |
| 7 | Hi-byte of X register |
| 8 | Lo-byte of X register |
| 9 | Hi-byte of Y register |
| 10 | Lo-byte of Y register |
| 11 | Hi-byte of U register |
| 12 | Lo-byte of U register |
| 13 | Value of MD register |
| 14 | Hi-byte of V register |
| 15 | Lo-byte of V register |
| 16 | Hi-byte of SP register |
| 17 | Lo-byte of SP register |
| 18 | Hi-byte of PC register |
| 19 | Lo-byte of PC register |
| 20-22 | Don’t Care |
| 23 | Checksum |

Table 25 – WireBug Write Registers Packet

Upon receipt of the packet, the CoCo shall compute the checksum of bytes 0-22 and compare its computed value against byte 23.  If a match occurs, the CoCo shall accept the packet’s payload, update its registers and send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | No error code (0) |

Table 26 – Checksum Match Packet

If a checksum match does not occur, then the CoCo shall disregard the packet and send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | Checksum mismatch (243) |

Table 27 – Checksum Mismatch Packet

Upon receipt of this packet, the Server may elect to restart the transaction.

Read Memory Transaction

This bi-directional transaction is sent from the Server to the CoCo.  The Server uses this transaction to obtain memory values from the CoCo.

The following packet is sent from the Server:

| Byte | Value |
| --- | --- |
| 0 | OP_WIREBUG_READMEM ($4D) |
| 1 | Hi-byte of starting memory location |
| 2 | Lo-byte of starting memory location |
| 3 | Count (0 < n < 23) |
| 4-22 | Don’t Care |
| 23 | Checksum |

Table 28 – Read Memory Request Packet

Upon receipt of the packet, the CoCo shall compute the checksum of bytes 0-22 and compare its computed value against byte 23. If a checksum match does not occur, then the CoCo shall disregard the packet and send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | Checksum mismatch (243) |

Table 29 – Checksum Mismatch Packet

Upon receipt of the Checksum Mismatch Packet, the Server may elect to restart the transaction.

If a match occurs, the CoCo shall validate the memory request.  If byte 3 (count) is not between 1 and 22, then the CoCo shall send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | Illegal number (16) |

Table 30 – Illegal Number Packet

If the byte count is between 1 and 22, then the CoCo shall accept the packet’s payload and send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | OP_WIREBUG_READMEM ($4D) |
| 1..n | Memory from start to end |
| n+1..22 | Don’t care |
| 23 | Checksum |

Table 31 – Read Memory Response Packet

Upon receipt of the packet, the Server shall compute the checksum of bytes 0 to 22 and compare its computed value against byte 23.  If a match occurs, the Server shall accept the packet’s payload.  If a match does not occur, then the Server may elect to restart the transaction.

Write Memory Transaction

This bi-directional transaction is sent from the Server to the CoCo.  The Server uses this transaction to modify the contents of the CoCo’s memory.

The following packet is sent from the Server:

| Byte | Value |
| --- | --- |
| 0 | OP_WIREBUG_WRITEMEM ($6D) |
| 1 | Hi-byte of starting memory location |
| 2 | Lo-byte of starting memory location |
| 3 | Count (n bytes where 0 < n < 20) |
| 4-22 | Bytes to be modified |
| 23 | Checksum |

Table 32 – Write Memory Request Packet

Upon receipt of the packet, the CoCo shall compute the checksum of bytes 0-22 and compare its computed value against byte 23. If a checksum match does not occur, then the CoCo shall disregard the packet and send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | Checksum mismatch (243) |

Table 33 – Checksum Mismatch Packet

If a match occurs, the CoCo shall validate the memory request.  If byte 3 (count) is not between 1 and 19, then the CoCo shall send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | Illegal number (16) |

Table 34 – Illegal Number Packet

If the byte count is between 1 and 19, then the CoCo shall accept the packet’s payload, update its memory with the contents from the Server, and send the following response packet:

| Byte | Value |
| --- | --- |
| 0 | No error code (0) |

Table 35 – Checksum Match Packet

Continue Execution Transaction

This uni-directional transaction is sent from the Server to the CoCo.  The Server uses this transaction to notify the CoCo that it should continue executing normally.

The following packet is sent from the Server:

| Byte | Value |
| --- | --- |
| 0 | OP_WIREBUG_GO ($47) |
| 1-23 | Don’t care |

Table 36 – Continue Execution Packet

Upon receiving this packet, the CoCo shall exit WireBug Mode and commence execution.  The Server shall commence responding to CoCo-initiated DriveWire transactions.

Virtual Serial Ports

DriveWire 4 introduces a special set of transactions to support virtual serial ports.

Serial Read

Using this bi-directional transaction, the CoCo asks the Server for any pending serial data bytes that may be pending.

| Byte | Value |
| --- | --- |
| 0 | OP_SERREAD ($??) |

Table 37 – Serial Read Request Packet

Upon receipt of this packet, the server shall return the following response packet:

| Byte | Value |
| --- | --- |
| 0 | bits 0-1: 00 - normal serial read response, either nothing waiting or a single byte for port specified in lowest 3 bytes 01 - status change for port, new status is data byte 10 - currently unused 11 - multiread request bit 2: 1 - Server would like CoCo to send another OP_SERREAD request immediately after processing this one. CoCo is not required to adhere to this request. bits 7-5: port number (0-7) |
| 1 | bits 0-1: 00 - data byte for port 01 - status byte for port 11 - number of bytes waiting on server |

Table 38 – Serial Read Request Response Packet

Serial Get Status

Using this bi-directional transaction, the CoCo asks the Server for the status of a particular port.

| Byte | Value |
| --- | --- |
| 0 | OP_SERGETSTAT ($??) |
| 1 | bits 2-0: port number to request status for |

Table 39 – Serial Get Status Packet

Upon receipt of this packet, the Server shall return the following response packet:

| Byte | Value |
| --- | --- |
| 0 |  |
| 1 |  |

Table 40 – Serial Get Status Response Packet

Serial Set Status

Using this bi-directional transaction, the CoCo sets the status for a particular port.

| Byte | Value |
| --- | --- |
| 0 | OP_SERSETSTAT ($??) |
| 1 | bits 4-0: 00000 - open port 11111 - close port |

Table 41 – Serial Set Status Packet

Upon receipt of this packet, the Server shall return the following response packet:

| Byte | Value |
| --- | --- |
| 0 |  |
| 1 |  |

Table 42 – Serial Set Status Response Packet

Serial Read Multiple

Using this bi-directional transaction, the CoCo requests the reading of multiple bytes in one call.

| Byte | Value |
| --- | --- |
| 0 | OP_SERREADM ($??) |
| 1 | port number |
| 2 | number of bytes requested |

Table 43 – Serial Read Multiple Packet

Upon receipt of this packet, the Server shall return the following response packet:

| Byte | Value |
| --- | --- |
| 0..n-1 | Data bytes returned |

Table 44 – Serial Set Status Response Packet

Appendix A. Error Codes

The error codes used by the Server to indicate an error condition mirror the same codes used in OS-9.  Here is a list of codes that should be used and the conditions that will trigger them:

- $F3 – CRC Error (if the Server’s computed checksum doesn’t match a write request from the CoCo)

- $F4 – Read Error (if the Server encounters an error when reading a sector from a virtual drive)

- $F5 – Write Error (if the Server encounters an error when writing a sector)

- $F6  - Not Ready Error (if the a command requests accesses a non-existent virtual drive)

Appendix B. DriveWire Checksum Algorithm

Even though DriveWire has been shown to provide very stable and reliable communications between CoCo and Server, a checksum algorithm has been employed to bring an added level of data integrity.

This new checksum algorithm is simpler and significantly better at tracking bit errors than the CRC algorithm employed in DriveWire Version 1, which used a fast but weak CRC method.

Designers of DriveWire Server software should implement the following checksum algorithm (presented in C source code) for computing a checksum for all sectors read and written to/from the CoCo.

int calChecksum(char *ptr, int count)

{

short Checksum = 0;

while(--count)

{

Checksum += *(ptr++);

}

return (Checksum);

}

Note that WireBug uses an 8-byte checksum instead of a 16 bit one.